# 4     Array Management

This chapter describes how to manage a disk array system.  Topics covered include the following:

- Background information about disk array management

- Setting controller configuration

- Setting channel configuration

- Scanning in drives

- Creating and deleting logical drives

- Adding and deleting spare drive assignments

- Rebuilding logical drives

- Creating and deleting logical volumes

- Creating and deleting volume partitions

- Mapping logical volumes or partitions to host LUNs

- SAF-TE / I2C / Fault Bus

- Defining enclosures

- Displaying the contents of the event log

- Monitoring performance statistics

- Dynamic logical drive expansion

- Redundant controller use

If this is your first time to manage a disk array system, we recommend that you read through section 4.1 Background Information, to get basic information about disk array management.  You will need this basic knowledge to be able to effectively use RAIDWatch Manager.

## 4.1     Background Information

Redundant Arrays of Independent Disks (RAID) is a storage technology used to improve the processing capability of storage systems.  This technology is designed to provide reliability (i.e., “fault tolerance”) in disk array systems and to take advantage of the performance gains multiple disks can offer.

RAID comes with a redundancy feature that ensures fault-tolerant, uninterrupted disk storage operations.  In the event of a disk failure, disk access will still continue normally with the failure transparent to the host system.

RAID has six levels:  RAID 0 ~ 5.  RAID levels 1, 3 and 5 are the most commonly used levels, while RAID levels 2 and 4 are less popular.  Appendix D, RAID Levels, gives information about these levels, including the benefits of each.

Infortrend disk array controllers support hot-swapping where a failed drive can be replaced while the disk array system continues to function.  Spares can also be assigned so that, as soon as a drive fails, the spare will be automatically configured into the array and reconstruction will commence.

### 4.1.1	Definition of Terms

- This section describes some of the disk array terms used in this documentation.

- Physical drives.  These are the actual SCSI drives installed on the connectors of the SCSI cables.  These drives are displayed in Physical View under the RAID View window.

- Spare drives.  These are physical drives that serve as backups.  When a drive fails, the spare is automatically configured into the array, and data reconstruction will commence immediately.  Spare drives appear in darker (shaded) colors than normal drives and have a red cross superimposed on them.  Large red crosses indicate Global spares, smaller ones represent Local spares.

- Replacement drives.  These are physical drives that are manually configured into the array to replace failed drives.  In the absence of spare drives, you will need to use replacement drives to replace defective drives before rebuilding.  If a spare drive has been used to rebuild the array, you will also need to replace the failed drive manually to create another spare with the precaution that another drive might fail.

- Failed drives.  These are physical drives that fail due to some type of error.  Failed drives appear with large red X marks on their respective icons.

- Logical drives.  These drives are created using physical drives.  Combining physical drives into logical drives gives you a disk array with a certain RAID level.  To view logical drives, use Logical View under the RAID View window.

- Logical volumes.  These volumes are created using logical drives.  Combining logical drives into logical volumes gives you a single logical unit with even larger capacity.  Logical volumes or their partitions are mapped to various host LUNs.  To view logical volumes, use Logical View under the RAID View window.

### 4.1.2	Operating With Spare Drives

You can assign spare drives to a particular logical drive to serve as backup drives.  When a drive fails within the logical drive, one of the spares will be automatically configured into the logical drive, and data reconstruction onto it will immediately commence.

The following are guidelines for disk failure recovery when a spare drive is available:

- If a spare drive exists in the same logical drive, the controller will automatically mount the spare drive and start data rebuilding in the background.

- Depending on the design of the system external to the controller, it may be possible to remove a defective drive and replace it with a new drive without shutting down the system (hot-swapping).  Alternatively, the system can be shut down at a convenient time and the failed drive replaced.

- The replacement drive must then be assigned as a new spare drive.

## 4.2     Operating Without Spare Drives

The following are guidelines for disk failure recovery when a spare drive is not available:

- Depending on the design of the system, it may be possible to remove a defective drive and replace it with a new drive without shutting down the system (hot-swapping).  Alternatively, the system can be shut down at a convenient time and the system administrator can replace the failed drive.

- If the replacement drive is installed on the same channel and SCSI ID, you can then proceed with data rebuilding.

- If the replacement drive is installed on a different channel or SCSI ID, you need to scan in the new drive first then assign it as a spare drive of the logical drive which has had a drive failure.  Data rebuilding will have to be manually initiated.

IMPORTANT:  Although the RAID system provides uninterrupted disk access even after a disk failure, do not leave a failed drive unattended to.  Without replacement, the system will not survive a second physical drive failure on the same logical drive.  A defective drive must be promptly replaced and data rebuilt.

CAUTION: When performing hot-swapping, be sure to remove only the defective drive.  Removing the wrong drive will result in complete, unrecoverable data loss.  Use the Enclosure window or Physical View to locate exactly which physical drive has failed.

## 4.3     Before You Start

RAIDWatch Manager comes with password protection that prevents unauthorized modification of the disk array configuration.  During each attempt at modifying the system configuration, the configuration will be password protected.

By default, RAIDWatch Manager station comes without any password.  For information on how to set a password and other security features, see the Setting Up Security section of Chapter 3, Basic Operations.

## 4.4     Configuring the Controller

RAIDWatch Manager enables you to modify the configuration of the disk array controller from your manager console.  You can set caching optimization parameters of the system, set I/O queue limitations, set drive optimization parameters, set RAID verification options, get information about current system and board status, download firmware or NVRAM data to the controller, set redundant controller options, and modify the RAID system password among other variables.

The following is a complete list of configuration controls and information displays (more information about many of these variables is available in the controller hardware and firmware documentation).

### 4.4.1	Caching

- The data cache can be configured for optimal I/O performance using the following variables:

  - Caching Policy (choose one):

    - Cache Write Back.  (Default controller setting.)  Recommended operating mode, provides better performance.

    - Cache Write Through.  Used primarily if no cache battery backup is installed and there is increased likelihood of a power failure.

  - Optimization Policy  (choose one):

- IMPORTANT:  Optimization settings should not be changed after logical drives are created. Under some circumstances, changing the optimization setting after logical drives have been created will destroy existing data on those drives.

    - Optimization for Random I/O.  More common setting.  Use this option for environments (e.g., database maintenance) with smaller I/O transactions.

    - Optimization for Sequential I/O.  Used for large I/O environments such as video recording and editing.  Particularly useful where I/O read/write must be in sequential order.

### 4.4.2	Host-Side

  - Host-side SCSI Parameters (choose from the range):

    - Maximum Queued I/O Count.  Allows you to control the maximum size of the I/O queue.  Available size selections: Auto, 1, 2, 4, 8, 16, 32, 64, 128, 256, 512, 1024.

### 4.4.3	Drive-Side

  - Drive Side SCSI Parameters (choose from each range):

    - Power-Up SCSI Reset.  Choose Enabled or Disabled.

    - SCSI Motor Spin-Up.  Choose Enabled or Disabled.  Enabled causes the controller to spin-up the SCSI drives at a spaced time interval rather than all at once.

    - Maximum Tag Count.  Available selections: Disable, 1, 2, 4, 8, 16, 32, 64, 128.

    - Disk Access Delay Time (Sec.).  Available selections: No Delay, 5, 10, 15, 20. . .75.

    - SCSI I/O Timeout (Sec.).  Available selections: Default(7.0), 0.5, 1, 2, 4, 6, 8, 10.

    - Drive Check Period (Sec.).  Available selections: Disabled, 0.5, 1, 2, 5, 10, 30.

    - SAF-TE Device Check Period (Sec.).  Available selections: Disabled, 0.05, 0.1, 0.2, 0.5, 1, 2, 5.

    - Drive Fail Swap Check Period (Sec.).  Available selections: Disabled, 5, 10, 15, 30, 60.

    - SMART.  Available selections: Disabled, Detect Only, Clone Only, Clone&Replace.

- Detect: detect and report SMART errors

- Clone Only: clone drive data to spare automatically when an error is detected

- Clone&Replace: clone drive data to spare and replace drive with spare when cloning is finished

### 4.4.4	RAID

  - Disk Array Parameters (choose from each range):

    - LD Rebuild Priority.  Available selections: Low, Normal, Improved, High.

    - Write Verify On LD Initialization.  Choose Enabled or Disabled.

    - Write Verify On LD Rebuild.  Choose Enabled or Disabled.

    - Write Verify On Normal Drives Access.  Choose Enabled or Disabled.

### 4.4.5	Controller

  - Information (no configurable parameters, readouts only)

    - System: Total Cache Size, Firmware Version, Bootrecord Version, Serial Number, CPU Type.

    - Board:  CPU Temp Sensor, Board Temp Sensor, +3.45V Value, +5V Value, +12V Value

  - Operation (choose from each range or enter a value):

    - Name:  A user-configurable identifier for the controller.  Note, the name and password fields combined have a maximum size of 16 characters.

    - Password:  A user-configurable security setting.  Note, the password and name fields combined have a maximum size of 16 characters.

    - Download:  Choose Download Firmware, Download Firmware/Bootrecord, Download NVRAM, Upload NVRAM, Download BIOS.  All of the Download functions will prompt for a file source from the current workstation.  Upload NVRAM will prompt for a file destination at the current workstation.

    - System:

      - Mute Beeper – temporarily mutes the controller beeper if it is currently sounding;

      - Reset Controller – resets the controller (similar to a PC reset), allowing configuration changes to take effect.

    - Redundant:

      - Controller Unique Identifier – This unique ID is used by controller to generate a controller-unique WWPN.  WWPN is a Fibre channel port name.  If redundant controller configuration is preferred and host interface is Fibre channel.  Each controller in redundant controller configuration MUST be assigned with a unique ID from 1 to 65535.

      - Redundant Controller Configuration – Enabled or Disabled depending on whether or not the current RAID has controller redundancy;

      - Redundant Controller Channel – sets the communication channel for redundant controllers.

## 4.5     Configuring Channels

Using RAIDWatch Manager, you can modify the configuration of any channel on the controller.  You can set the channel operation mode to host or drive, enable or disable channel termination, set SCSI IDs, set the transfer clock rate for synchronous communication, and select the transfer width.

Channel configuration settings are available under Physical View in the RAID View window.

The following describes user-configurable channel parameters:

- Channel Mode.  This parameter sets the operation mode of a particular channel on the controller.  It configures the operation mode to either host or drive, where a host channel is used when connecting to a host and a drive channel is for connecting to SCSI drives.  Any SCSI channel on the controller can serve as either a host or drive channel.  Channel 0 is the default host channel with SCSI ID number 0.  In redundant mode, channel 0 is also the default channel for synchronized cache communiations.

- Termination.  This parameter enables or disables SCSI channel termination.  SCSI cables must be properly terminated at both ends; that is, when connecting one end of a SCSI cable to a channel, the termination of the channel must be enabled.  Note that the other end of the cable must also be properly terminated.

You can terminate the other end of the cable by installing or enabling termination on the SCSI drive farthest from the controller or by installing an external terminator on the end connector.  The latter method is recommended so that removal of the drives will not affect cable termination.  All other terminators must be removed or disabled making sure that only one terminator is installed on each end.  (SentinelRAID series users, check your hardware documentation for additional termination information.)

- Default Xfer Clock (in MHz), Default Xfer Width.  These parameters set the data transfer clock rate for synchronous communication over the SCSI bus, and enable or disable wide transfer, respectively.

Data that is transferred across the SCSI bus in synchronous transfer mode is clocked using a synchronous transfer clock signal.  The frequency of this signal determines the rate at which data is transferred.  If, for example, the synchronous transfer clock is 10 MHz, data transfer rate will be 10 million bytes per second (assuming narrow transfer).

Data transfer across the SCSI bus can be either 8 bits or 16 bits at a time.  The former is referred to as narrow transfer, while the latter is referred to as  wide transfer.  At the same synchronous transfer clock rate, data transfer using wide transfer will be double that of narrow.  With a transfer clock of 10MHz, the data transfer rate will be 10 Mbytes/second under narrow transfer, and 20Mbytes/second using wide.

Occasionally, under conditions in which SCSI signal quality is poor, such as with extremely long cables, poor connections, or bad termination, it may be necessary to reduce the synchronous transfer clock to allow the SCSI channel to function normally.  Worst case, it may be necessary to switch to asynchronous communication mode.

Furthermore, certain older SCSI devices may only support narrow transfer and behave abnormally when attempting to negotiate wide transfer.  Under such conditions, wide transfer may need to be disabled, forcing the controller to use narrow transfer mode with that device.

- ID pool / PID / SID.  This parameter sets the SCSI ID of the channel.  Each channel must have a unique SCSI ID in order to work properly.  SCSI ID ranges from 0 up to 15, with 0 assigned as the default value for host channels, and 7 for drive channels.

- It is necessary to create a SID on every I/O channels in redundant controller configuration.  The default are 6 for SID and 7 for PID on drive channels.  The default values for Infortrend's dual- redundant configuration are 8 for SID and 9 for PID.  For more information, please refer to the hardware documentation that came with your controller.

 Setting the configuration of a channel

1.	Display Physical View by clicking on the Physical View command button under the RAID View introduction or by selecting Physical View from the RAID View navigation panel.

2.	From Physical View, double-click on the corresponding SCSI Channel icon of the target channel.  SCSI Channel icons are displayed in the navigation panels on the left side of the RAID View window. The Channel Settings configuration will appear in the RAID View content panel.

3.	If you are using a SCSI-to-SCSI controller, select the channel mode from the Channel Mode drop-down list box.  To select, click on the down-arrow button at the right of the box, then select the option you want.  If you want to configure the selected channel as a host channel, select Host; to configure as a drive channel, select Drive.

4.	From the Termination drop-down list box, specify whether to enable the channel termination or not.  To enable, select On; to disable, select Off.  (IFT-3200 series users, check your hardware documentation for additional termination information.)

5.	If you want to assign a different SCSI ID to the selected channel, choose the new ID from the ID pool scroll list box.  When selecting an ID, be sure that it does not conflict with the other SCSI devices on the channel.  The ID pool lists all of the channels.  Highlight the ID you want to use and click Add next to either the PID (Primary ID) or SID (Secondary ID) window.

6.	To change the speed of the SCSI bus synchronous transfer clock, choose the new value from the options listed in the Default Xfer Clock drop-down list box.

7.	From the Default Xfer Width drop-down list box, select whether to use narrow or wide transfer.  To use narrow transfer, select Narrow; to use wide transfer, select Wide.

8.	Click OK to save the new channel configuration into the non-volatile memory (NVRAM) of the controller.  You will be prompted for a password in order to complete the changes.

### 4.5.1	Configuring New or Unassigned Drives

New and unassigned drives have some additional configuration options listed by channel under Physical View.

  - Drive Info (no configurable parameters, readouts only)

    - Drive Information: Slot Number, Channel Number, ID Number, Capacity, Status, Transfer Rate, Logical Drive ID, Vender/Product ID.

  - Operation

    - Copy&Replace:  used to copy and replace a current member drive of a logical drive.  Choose the target drive by clicking on its icon in the Copy& Replace window pane.  Click OK to replace the target with the selected new drive.

- NOTE: This operation is intended for use in expanding a logical drive by replacing all member drives one at a time.  Also, as the available size will be limited to the maximum size of the smallest hard drive, it is recommended that all drives in a logical drive be the same size.

  - Scan New Drive

- Choose the channel number and ID where the new drive is located.

- Click OK.  The system will ask for a password and then scan for a drive with the specified ID.  It will then either display a successful confirmation message or an error message.

- Repeat these steps to scan additional drives.

  - Add Spare

- Choose a logical drive to which to add a spare.  Once it is displayed in the Target LD text box, click the Local button (to make a local spare) or the Global button (to make a global spare).

## 4.6     Scanning in Drives

Disk scanning is a process in which a newly installed drive is introduced to the disk array system.  You need to scan in a drive when:

- The drive is a new one.  A drive is considered a new drive if it is not yet displayed in the Physical View display of the RAID View window.

- The drive is a replacement for a failed drive which is installed on a different channel or has a different SCSI ID.

If you do not scan in the drive on such occasions, the system will not be able to see the drive, rendering it useless.

 Scanning in a drive

1.	Display Physical View under the RAID View window by clicking on the Physical View command button under the introduction or selecting Physical View from the navigation panel.

2.	In Physical View, double-click on the channel corresponding to the SCSI ID of the drive to be scanned.

3.	Click on a drive icon and choose Scan New Drive.

4.	Choose the channel number and ID where the new drive is located.

5.	Click OK.  The system will ask for a password and then scan for a drive with the specified ID.  It will then either display a successful confirmation message or an error message.

6.	Repeat steps 2 through 5 to scan additional drives.

RAIDWatch Manager will add a drive icon to the relevant channel if the scanning process is successful.  The new drive can now be added to an existing logical drive or used to create a new logical drive.

## 4.7     Creating and Deleting Logical Drives

Logical drives can be created by combining physical drives together.  Under Logical View in the RAID View window, click Logical Drives.

RAIDWatch Manager also allows you to delete logical drives.  Before deleting, however, be sure that the data stored on the target logical drive is no longer needed.  Deleting a logical drive erases all information stored on that logical drive.

NOTE:  When you delete a logical drive, all physical drives assigned to the logical drive will be released, making them available for creation or expansion of logical drives.

 To create a logical drive

The contents panel will display Create Logical Drive and show any available (i.e., unassigned) physical drives organized by channel.  Select the drives you want to include in the logical drive and click Add or click Add All. Once drives have been added to the “Selected” window, available RAID levels will be listed under RAID parameters.

1.	Display the Logical Drives window by clicking on the Logical View command button in the introduction or selecting Logical View from the RAID View navigation panel.

2.	From Logical View, click on Logical Drives in the navigation panel, then click on a channel listed under “ Create Logical Drive / Drives Selection / Available.”  Select the drives you want to include in the logical drive and click Add or click Add All to select all currently unassigned drives.

Once drives have been added to the “Selected” window, available RAID levels will be listed under RAID parameters.

3.	Select a RAID Level from the RAID Level menu.

4.	Enter a Maximum Drive Size in that text box.  The value entered determines how much capacity from each drive will be used in the logical drive.  Unused drive capacity can then later be used to expand the logical drive.

5.	Choose Primary Controller or Secondary Controller from the LD Assignment menu.  If redundant controller function has not enabled or that SID's are not assigned on drive channels, LD Assignment menu will not be available.

6.	Click OK.  You will be prompted for the controller password.  Enter the password and click OK.

7.	The Create In Progress content window will display logical drive creation progress.

 To expand a logical drive

When physical drives are added to the array, the additional capacity can be added to existing logical drives.

1.	Display the Logical Drive window by clicking on the Logical View command button in the introduction or selecting Logical View from the RAID View navigation panel.

2.	From Logical View, click on Logical Drives and choose the logical drive where you would like to expand capacity.

3.	Choose the Operation tab and the Expand sub-tab, then note whether or not there is a number in the Maximum Free Capacity (MB): text box.

4.	If there is an amount in the Maximum Free Capacity text box, you may enter an amount in the Set Expansion Size (MB): text box up to the total of free capacity.

5.	Click OK.  You will be prompted for the controller password.  Enter the password and click OK.

6.	The logical drive will now have a new last partition the same size as the expansion.  Look at the View and Edit LD Partition Table to verify this.

NOTE:  See section 4.17 for a detailed explanation of dynamic logical drive expansion principles.

-  To delete a logical drive

1.	Display the Logical Drives window by clicking on the Logical View command button in the introduction or selecting Logical View from the RAID View navigation panel.

2.	From the Logical Drives window, click the left mouse button on the target logical drive.

3.	Select the Operation tab.

4.	Select the Delete sub-tab, click on the logical drive to delete, and click OK.  You will be prompted for a password.  Enter it and click OK.  The logical drive will be deleted and removed from the logical drives list.

IMPORTANT: Deleting a logical drive irretrievably wipes all data currently stored on the logical drive.

## 4.8     Adding and Deleting Spare Drive Assignments

You can assign spare drives to a logical drive to serve as backups for failed drives.  In the event of a drive failure, the spare drive will be automatically configured into the array and reconstruction (or rebuilding) will immediately commence.

Logical drives can support multiple spare drives; this configuration however is rarely used due to its high cost and uncommon occurrences of drive failures. A practical configuration calls for one spare drive per logical drive  after rebuilding on this drive, just replace the failed drive and then configure the replacement as the new spare drive of the logical drive.

NOTE:  Adding a spare drive can be done automatically by selecting the RAID 1+Spare,  RAID 3+Spare or RAID 5+Spare option from the logical drive RAID Level selection dialog box when creating a logical drive.  These options apply to RAID 1, RAID 3, and RAID 5 levels respectively.

 Assigning a spare drive

1.	Display the Logical Drives window by clicking on the Logical View command button in the introduction or selecting Logical View from the RAID View navigation panel.

2.	Choose the logical drive where you would like to assign a spare drive.  Choose the Operation tab and the Spare sub-tab.  If the Spare sub-tab does not appear, there are no drives available to serve as spares.

3.	Under the Add tab, if there are drives available to assign as spares, their channels will be listed under Available.  Choose the drive icon for the drive that will become the new spare.

4.	Click the Add button, then click either Local or Global.  Local spares will only replace failed drives in the same logical drive.  Global spares will replace any drive that fails in any logical drive on the RAID controller.

NOTE: Spare drive assignments can also be made when scanning in new drives under Physical View.

 Deleting a spare drive assignment

1.	Display the Logical Drives window by clicking on the Logical View command button in the introduction or selecting Logical View from the RAID View navigation panel.

2.	Choose the logical drive where you would like to delete an existing spare drive assignment.  Choose the Operation tab and the Spare sub-tab.

3.	Under the Delete tab, if there are drives assigned as spares, their icons will be displayed under Spare Drives.  The upper window under Spare Drives lists Local Spares.  The lower window lists Global Spares.  Choose the drive icon for the drive that you want to delete and click Add.

4.	Spare drives to delete will be added to the Selected window.  Once all of the spare drives you want to delete are listed, click Delete.  You will be prompted for a password.  Enter it and click OK.  All selected spare drives will return to normal status.

## 4.9    Rebuilding Logical Drives

Depending on whether or not there is a spare drive, rebuilding is initiated automatically or must be started manually.  In the presence of a spare drive, the system automatically rebuilds onto the spare drive.  This process is done in the background, thus it is transparent to users.

In the absence of a spare drive, rebuilding must be initiated manually.  Before initiating a manual rebuild, you must first replace the failed drive.  If you install the replacement drive on the same connector (that is, the same channel and SCSI ID), then you can proceed with the rebuilding process; otherwise, you need to scan in the drive first.

 To rebuild a logical drive

1.	Display the Logical Drives window by clicking on the Logical View command button in the introduction or selecting Logical View from the RAID View navigation panel.

2.	In the Logical Drives window, click on the degraded logical drive.

3.	Choose the Operation tab and then the Rebuild sub-tab.

4.	Choose a logical drive to rebuild.  Logical drives in need of rebuilding will be marked “degraded” and will have darkened icons.  Click OK.  You will be prompted for a  password.  Enter it and click OK.

5.	The Rebuild in Progress window will appear and show the percentage of rebuild progress until complete.

6.	Once the logical drive has been rebuilt, the logical drive icon will return to its normal color and the Rebuild tab will no longer be available under Operation.

## 4.10    Creating and Deleting Logical Volumes

You can create and delete logical volumes using RAIDWatch Manager.  Logical volumes are created by combining logical drives together.  You can combine logical drives with different capacities and RAID levels into a single logical volume.  You can also delete existing logical volumes.  Before deleting, make certain that the data stored in the logical volume is no longer needed.  Deleting a logical volume erases all information stored on that logical volume.

NOTE:  When you delete a logical volume, all logical drives assigned to it will be released, making them available for new logical volume creation.

 To create a logical volume

The contents panel will display Create Logical Volume and show any available (i.e., unassigned) logical drives organized by ID.

1.	Display the Logical Volume window by clicking on the Logical View command button in the introduction or selecting Logical View from the RAID View navigation panel.

2.	From Logical View, click on a channel listed under “LDs Selection / Available.”  Select the drives you want to include in the logical volume and click Add or click Add All to select all currently unassigned drives.

3.	The Maximum LD Capacity (MB) is an information only text box.  It is not a user-configurable value.  The figure listed indicates the maximum capacity of each LD that will be used in the new LV.

4.	Choose Primary Controller or Secondary Controller from the Logical Volume Assignment menu.

5.	Click OK.  You will be prompted for the controller password.  Enter the password and click OK.

6.	The new logical volume will be displayed in the navigation panel.  You may now define its partitions.  See the Creating and Deleting Partitions section of this chapter.

 To expand a logical volume

When logical drives are expanded by adding new drives, additional capacity can be added to existing logical volumes.

1.	Display the Logical Volume window by clicking on the Logical View command button in the introduction or selecting Logical View from the RAID View navigation panel.

2.	From Logical View, click on Logical Volumes and choose the logical volume which has an expanded capacity logical drive.

3.	Choose the Operation tab and the Expand sub-tab, then note whether or not there is a number in the Maximum Free Capacity (MB): text box.

4.	If there is an amount in the Maximum Free Capacity text box, you may enter an amount in the Size to Expand (MB): text box up to the total of free capacity.

5.	Click OK.  You will be prompted for the controller password.  Enter the password and click OK.

6.	The logical volume will now have a new last partition the same size as the expansion.  Look at the View and Edit LV Partition Table to verify this.

NOTE:  You may combine partitions under View and Edit LV Partition Table by expanding the size of earlier partitions (as in, increase the size of partition 0 so that it is as large as all partitions combined to make one partition).  Combining partitions destroys existing data on all drive partitions.

-  To delete a logical volume

1.	Display the Logical Volumes window by clicking on the Logical View command button in the introduction or selecting Logical View from the RAID View navigation panel.

2.	From the Logical Volumes window, click the left mouse button on the target logical volume.

3.	Select the Operation tab.

4.	Select the Delete sub-tab, click on the logical drive to delete, and click OK.  You will be prompted for a password.  Enter it and click OK.  The logical drive will be deleted and removed from the logical drives list.

IMPORTANT: Deleting a logical drive irretrievably wipes all data currently stored on the logical drive.

## 4.11    Creating and Deleting Partitions

Partitions can be created in both logical drives (LD) and logical volumes (LV).  Depending on your specific needs, you can partition an LD or LV into smaller sizes or just leave it at its default size (that is, one large partition covering the entire LD or LV).

If you intend to map an entire LD or LV to a single host LUN, then partitioning becomes irrelevant.  If, however, the LD or LV is to be mapped to multiple host LUNs, then you need to define partitions for later LUN  mapping.

NOTE:  You can create a maximum of eight partitions per logical drive or logical volume.  Also, partitioned logical drives can not be part of a logical volume.

 Partitioning a logical drive (LD)

1.	Display Logical Drives by clicking on the Logical View command button in the introduction or selecting Logical View from the RAID View navigation panel.

2.	From Logical View, choose Logical Drives and then the LD that will be partitioned.  After you select an LD, the contents panel will display the current View and Edit LD Partition Table.

3.	To create a new partition, edit an existing one, or remove an existing partition, click the Edit button.  You will be prompted for a password, enter it and click OK.

4.	Select partition 0 or the last partition in the list to create a new partition.  Only the Size(MB) field is editable and partitions must be contiguous.  Modify the value of partition 0 or the last partition so that it will be the size you intend.  All unallocated space will automatically be assigned to the new partition.

NOTE:  Any partition can be modified using the method described here.  Changes will be reflected in other partitions as disk space is occupied or freed.

5.	If you want to make multiple partitions, repeat the process.  If you exit the View and Edit LD Partition Table, you will need to click Edit and enter a password again before you can make changes.

6.	To remove partitions, increase the size of partitions higher in the list.  Last partitions will always be absorbed first.

The logical drive is now ready for mapping to host LUNs. See: Mapping Logical Drives/Volumes/Partitions to Host LUNs.

 Partitioning a logical volume (LV)

1.	Display Logical Volumes by clicking on the Logical View command button in the introduction or selecting Logical View from the RAID View navigation panel.

2.	From Logical View, choose Logical Volumes and then the LV that will be partitioned.  After you select an LV, the contents panel will display the current View and Edit LV Partition Table.

3.	To create a new partition, edit an existing one, or remove an existing partition, click the Edit button.  You will be prompted for a password, enter it and click OK.

4.	Select partition 0 or the last partition in the list to create a new partition.  Only the Size(MB) field is editable and partitions must be contiguous.  Modify the value of partition 0 or the last partition so that it will be the size you intend.  All unallocated space will automatically be assigned to the new partition.

NOTE:  Any partition can be modified using the method described here.  Changes will be reflected in other partitions as disk space is occupied or freed.

5.	If you want to make multiple partitions, repeat the process.  If you exit the View and Edit LV Partition Table, you will need to click Edit and enter a password again before you can make changes.

6.	To remove partitions, increase the size of partitions higher in the list.  Last partitions will always be absorbed first.

The logical drive is now ready for mapping to host LUNs. See: Mapping Logical Drives/Volumes/Partitions to Host LUNs.

## 4.12    Mapping Logical Drives/Volumes/Partitions to Host LUNs

After creating a logical drive (LD) or logical volume (LV), you can map it as is to a host LUN; or, if partitions are set, you can map each partition to a specific host LUN.  RAIDWatch supports 8 LUNs per host channel (numbered 0 - 7), each of which appears as a single drive letter to the host if mapped to an LD, LV, or a partition of either.

Existing host LUN mappings also can be deleted.  In cases where certain mappings are found to be useless, or disk array reconfiguration is needed, you can delete unwanted mappings in your system.

 Map an entire logical drive or volume to a host LUN

1.	First, be certain that the logical drive or logical volume to map is not partitioned.

2.	In the navigation panel under Physical View, click on the host channel where you would like to map a drive or volume.

3.	In the sub-navigation window, click on a host channel ID number.  The LUN Table for the host channel will be displayed.

4.	Choose the LUN Mapping tab, and under the Add sub-tab, choose either LD (for logical drives) or LV (for logical volumes).

5.	List the available logical volumes or logical drives by clicking on the “All” icon in the respective window.  Click on the drive or volume you would like to map to display its partition 0. Partition 0 should be the only partition listed if you want to map an entire drive or volume to a single LUN.

6.	From the LUN Selected drop-box, choose the LUN ID to assign.  Only unused IDs will be listed.

7.	Next, click on the partition 0 icon.  “Partition 0,” should appear in the Partition Selected box. Click OK and enter the password when prompted.

8.	If you choose the LUN Table, the newly mapped LUN should now be listed.

 Map a logical drive or volume partition to a host LUN

1.	First, partition the logical drive or logical volume (see section 4.11 above).

2.	Once the drive or volume has been partitioned, in the navigation panel under Physical View, click on the host channel where you would like to map partitions.

3.	In the sub-navigation window, click on a host channel ID number.  The LUN Table for the host channel will be displayed.

4.	Choose the LUN Mapping tab, and under the Add sub-tab, choose either LD (for logical drive partitions) or LV (for logical volume partitions).

5.	List the available logical volumes or logical drives by clicking on the “All” icon in the respective window.  Click on the drive or volume you would like to map to display its partitions. Each drive and volume can have up to 8 partitions, each of which can be mapped to a host LUN.

6.	From the LUN Selected drop-box, choose the LUN ID to assign.  Only unused IDs will be listed.

7.	Next, double-click on the icon of the partition you would like to map.  The partition should appear in the Partition Selected box.  Click OK and enter the password when prompted.

8.	Repeat steps 6 and 7 to map additional partitions.

9.	Once all partitions have been mapped, if you choose the LUN Table tab, the newly mapped LUNs should be listed.

 Deleting a host LUN mapping

1.	In the navigation panel under Physical View, click on the host channel where you would like to delete a host LUN mapping.

2.	In the sub-navigation window, click on a host channel ID number.  The LUN Table for the host channel will be displayed.

3.	Choose the LUN Mapping tab, and then the Delete sub-tab.

4.	To delete a LUN mapping, double-click on it in the LUN Map table until the ID is displayed in the LUN Selected box, then click OK.  When prompted for a password, enter it and click OK.

5.	The LUN mapping should no longer be listed in the LUN Map table.  Repeat step 4 to remove additional LUN mappings.

## 4.13    SAF-TE / I2C / Fault Bus

Support for SAF-TE, I2C, and Fault Bus devices and monitoring allows RAIDWatch to display current status for SAF-TE and I2C, and provides status displays and some minimal configuration of Fault Bus.

To display SAF-TE, I2C, or Fault Bus, under Physical View in the navigation panel, choose the icon for the one you want.  For SAF-TE and I2C, you must then double-click on  the specific device in the sub-navigation panel.

### 4.13.1	 SAF-TE and I2C

- Both SAF-TE and I2C are enclosure-proprietary monitoring technologies which must be configured by the enclosure manufacturer.

RAIDWatch is capable of displaying any information provided by a SAF-TE or I2C device.  Included among the kinds of information typically provided is status for:

- Power Supplies

- Fans

- Ambient Temperature

- UPS

Click on the appropriate icon to display the status readout.

### 4.13.2   Fault Bus

- Access the Fault Bus by clicking on the Fault Bus icon in the navigation panel of Physical View.

To monitor the current status, click on the Fault Bus icon in the Monitor window.  The current status of the Power Supply, Fan, Temperature, and UPS should be displayed.

To modify the current Fault Bus settings, choose the Operation tab.  Under Operation, choose the Device tab to enable or disable each device monitor.

To change the active signal settings, choose the Signal tab under Operation, then select either High or Low for each device monitor.

To save any setting changes, click OK before configuring some other part of the RAID or the controller.

## 4.14    Defining Enclosures

There are two different cases with customized enclosures: Case 1 is an enclosure window pre-defined by system vendors.  The enclosure may be presented in a front view drawing or photograph.  The user may have to add devices into the pre-defined spaces.  Case 2 is an enclosure window showing only spaces representing device canisters.  In case 2, you have to start from the beginning to arrange devices in an enclosure.

Both cases allow you to replicate real enclosures with real drive bays, depicting the exact locations and positions of the physical drives and RAID controller(s).  In both cases, you create custom enclosures to facilitate management of the physical drives in the disk array system.  When a drive fails, determining which drive to replace will be simply a matter of checking the Enclosure window for the exact location of the failed drive (a failed drive appears with a red X mark on its icon).

 Creating an enclosure

1.	Display the Enclosure window.  To display it, click on the Enclosure command button (Case 1) or select the Enclosure command from the Open menu (Case 2).

- The Enclosure window has pre-configured spaces that resemble controller and drive canisters of your enclosure.  The enclosure window might have been defined by your system vendor.

- Click on an empty space that represents an empty canister or drive bay within the Enclosure window (horizontal or vertical).  The space you selected will be highlighted by different color.  Next click on a drive icon from the navigation panel, then click the Add button.  If an empty space does not match the drive icon or controller icon you selected, the Add button will be inactivated.  This happens when you try to place a drive in an inadequate canister (e.g. you can not place a drive in a controller canister; and you should arrange your drives according to actual locations and SCSI ID sequence).  Repeat to add more drives.  Using this same method, monitoring device icons can also be added to defined enclosures.

- Drives may only be added to one enclosure and the enclosure window provides no “auto-update” function.  If drives are added or removed from your array, you will need to update the enclosure settings in the Enclosure window.

- The Enclosure window also supports a logical view of connected drives.  Click on the Logical button to display the logical relationship among physical drives.  Physical drives configured in a logical drive group will be displayed in the same color.  If there are more than one logical drives, different colors will be displayed to distinguish members of different logical drives.

- Physical View under the RAID View window also provides a real-time report on drive status, using the same symbols and colors to represent various conditions.  What you see in the Enclosure window is also reflected in the Physical View.  These windows, however, differ in the way physical drives are presented; in the Enclosure window, the drives should be arranged according to their actual locations in the drive bays, while in the Physical View, the drives are arranged according to channel connections.

 Removing a drive from an enclosure

1.	Click on the target drive.

2.	Click the Remove button.

## 4.15    Displaying the Event Log

RAIDWatch Manager can display a running log of all events that occur in the disk array system ranging from simple notifications, to warnings, to alerts.

Note that the Event Log only displays events that occur while RAIDWatch Manager is running.

 To display the event log

1.	Click on the Event Log command button or select the Event Log command from the Open menu.  The Event Log window will appear

The events in the window are listed according to the date and time they occurred with the most recent event at the bottom.  A description of each event is  provided.

2.	To scroll through the list, use the vertical scroll bars.

NOTE:  If events occur and the Event Log window is not open, a red Event Flag button will appear on the button bar.  The Event Log can be accessed by clicking on this button.

## 4.16    Monitoring Statistics

RAIDWatch Manager includes a statistics monitoring feature to report on the overall performance of the disk array system.  This feature provides a continually updated real-time report on the current throughput of the system, displaying the number of bytes being read and written per second, and the percentage of data access being handled by cache memory.  These values are displayed in a graphical format.

To monitor current data traffic on the disk array

- Click on the Statistics command button or select the Statistics command from the Open menu.  Then choose either Cache Hits or Disk R/W (disk read/write).

The Cache Hits Statistics window displays what percentage of data is being accessed via cache memory. Read values appear in blue and Write values in red.

The Read/Write Statistics window displays the amount of data being read from and written to the disk array system, in MB per second.  Read values appear in blue and Write values in red.

## 4.17    Dynamic Logical Drive Expansion

What Is It and How Does It Work?

Before Dynamic Logical Drive Expansion, increasing the capacity of a RAID system using traditional methods meant backing up, re-creating and then restoring. Dynamic Logical Drive Expansion (a new feature in controller firmware versions 2.11 and later) allows users to add new SCSI hard disk drives and expand a RAID 0, 3 or 5 Logical Drive without powering down the system.

### 4.17.1  Two Modes of Dynamic LD Expansion

- There are two modes of Dynamic Logical Drive Expansion.

Mode 1 Expansion involves adding more SCSI hard disk drives to a logical drive, which may require that the user obtain an enclosure with more drive bays. The data will be re-striped onto the original and newly added disks.

In the figure above, new drives are added to increase the capacity of a 4-Gigabyte RAID 5 logical drive. The two new drives increase the capacity to 8 Gigabytes.

Mode 2 Expansion, on the other hand, requires the same number of higher-capacity SCSI hard disk drives for a given logical drive.

The figure above illustrates expansion of the same 4-Gigabyte RAID 5 logical drive using Mode 2 Expansion. Drives are copied and replaced, one by one, onto three higher-capacity drives.

This results in a new 4-Gigabyte, RAID 5 logical drive composed of three physical drives. The 4 Gigabytes of increased capacity is in a new partition.

IMPORTANT: The increased capacity from either expansion type will be a new partition.

Three new drives are scanned in (see section 4.6 for details on scanning in new drives.) To add the drives to the logical drive, select the logical drive where they will be added, then choose the Operation tab and Add Drive sub-tab.  Select a drive to add and click the Add button.  When you have selected all of the new drives you want to add, click OK.

The progress of the add process will be displayed as it is carried out.

The logical drive icon will appear to be degraded while the new drives are being added to the logical drive. The color will return to normal once the drive add is complete.

When you return to the partition table, you will notice that either partition 0 or the last partition will now be larger than before.

Follow the directions in section 4.12 to map the new partition to a host LUN. The new partition must be mapped to a host LUN in order for the HBA (host-bus adapter) to see it.

## 4.18    Redundant Controller

For SCSI-to-SCSI controllers, RAIDWatch Manager is capable of configuring some aspects of redundant controller implementation.

Under the Configuration window, choose the Controller tab, the Operation sub-tab, and the Redundant sub-sub-tab.  From this window pane, you can set the following redundant controller parameters:

- Controller Unique Identifier: enter a number  between 1 and 65535.

- Redundant Controller Configuration: from the drop-box, choose Disabled, Primary, Secondary, or AutoConfigure.

- Redundant Controller Channel: The is a hardware-dependent function.  Once the controller-controller negotiation is completed, this drop-box will display the interface used for communications.  Newer line of RAID controllers does not allow a switch between SCSI and RS-232.

All other Primary and Secondary redundant controller features must be configured using the controller’s front panel.  Please refer to your RAID controller instruction manual for details.