# A     ER2510FS RAID Controller Head

The ER2510FS RAID Controller Head is a high performance, 2Gb/s fibre-to-fibre storage management subsystem. It is a versatile system that comes with many features and enables user’s to configure subsystems that meet their particular needs. The latest RAIDWatch GUI version has added monitoring features that enable user’s to control and manage this storage subsystem. For more information on the ER2510FS Controller Head please refer to the Hardware Manual.

RAIDWatch provides user’s with an easy and efficient method of monitoring the controller head. A GUI customized for the ER2510FS Controller Head features a new Panel View that shows two interactive graphic representations, one of the front panel and one on the rear panel, of the physical system. The panel enables users to easily access information on the status of the controller head and its hardware components.

## Connecting to the RAID Agent

Connecting to the RAID Agent has been described in Chapter 3 of this manual. In brief, run the RAIDWatch program from the host computer. You will be asked to enter the IP address and the port number. Once the correct IP address and port number have been entered, a new screen will appear. The screen will have a directory tree structure. From the directory tree double-click on In Band SCSI. Once this has been selected you will be asked to enter the Controller Password. If you enter the correct password and are running an ER2510FS Controller Head, the Panel View screen will appear.

## Dual-Redundant Controllers

RAIDWatch is only able to manage one controller at one time on the ER2510FS RAID Controller Head. If the ER2510FS is in the dual redundant mode, i.e. running two controllers, RAIDWatch will only show the status of the one that is currently connected. If users want to see the status of the other controller, they have to run RAIDWatch again to manage it.

## The Panel View

The Panel View (see diagram) shows both the front panel and rear panel of the ER2510FS RAID Controller Head.

### Front View

The front view of the controller head shows the front panel of the ER2510FS. The front panel consists of a LCD panel, and 11 front panel modules. The first six slots on the front panel are reserved for IO modules, slot 7 for the LED module, slot 8 and 9 are reserved for the maintenance modules and slot 10 and 11 for the BBU modules. The modules that appear on the front panel depend on the ER2510FS model you are using (there are four models). Please refer to the Hardware Manual for further details.

Below the front view graphic is a section that describes the LED indicators on the LED module. The LED module is used to provide quick and easy controller information to the end user.

### LED Board Indicators

The LED Board Indicators are shown in the middle of the Panel View, between the Front and Rear View. The LED Panel, found in slot 7, contains two vertical LED arrays. The array on the left belongs to controller A and the array on the right to controller B. The LED Board Indicators correspond to the LED’s on the front panel.

### Rear View

The rear view of the controller head shows the rear panel of the ER2510FS. The rear panel consists of Power Supply Units (PSU) and cooling FAN modules. But, once again, due to the many different models that are available, the components that appear on the rear panel will depend on the ER2510FS model you are running. The rear panel is designed for two PSUs and two cooling FAN modules. In the above image, four fans appear on the rear panel. This is because each cooling FAN module consists of two individual fans.

## Accessing the Panel View

The Panel View should appear after you open the GUI and connect to the RAID Agent. If it doesn’t appear or you have closed the window but wish to re-access the Panel View you can either select “Panel View” from the “Open” menu on the top menu bar.

Alternatively selecting the Panel View Icon, which is circled in the figure below, can access the Panel View.

## Viewing System Information

The front view and rear view images that are seen in the Panel View are interactive and can be used to easily obtain system information. If the mouse is used to place the cursor on different components, a message tag will appear revealing the status and other information about the component.

### LCD Message

If you use the mouse to place the Cursor on the LCD Panel, a message like the one shown below will appear.

The message contains information on the Controller that “Owns” it. Please note that it is not possible to change controller ownership using the GUI. The following information is shown in the message tag:

Name: - This field shows the name that has been assigned to the controller.

ID: - This shows the ID that has been assigned to the controller.

CPU: - Shows the CPU that is installed on the controller.

Cache: - Shows the size and nature of the SDRAM chip that is installed for the cache memory.

FW: - This shows the Firmware version that has been installed on the controller.

BR: -  The Boot Record.

### IO Module

If a cursor is dragged over an IO module that is a Host Channel, the following message will appear

PID:- This reveals the primary ID of the channel.

SID:- This reveals the secondary ID of the channel.

IO Module LEDs:- Each IO module comes with two green LEDs. The top LED belongs to controller A and the bottom one to controller B. These LEDs indicate the speed of the fibre channel. If the LED is ON the speed of the fibre channel is 2Gb/s. If it is OFF then the speed is 1Gb/s. A flashing LED indicates loop link failure.

### LED Module

This is the only module on the front panel that does not come with a status message tag. The LED’s on the LED Module shown in the Panel View reflects the status of the LED’s on the physical module, i.e. if the top LED is for controller A on the Controller Head is “ON”, it will appear in the Panel View as “ON”.

The LED Indicators are specified in the center of the Panel View, between the Front Panel and Rear Panel views. The following list specifies the function of each LED in a single array.

LCD Owner:- If the top LED in the array on the left is  “ON” then the information shown on the LCD screen relates to controller A. If the top LED in the array on the right is  “ON” then the information shown on the LCD screen relates to controller B.

Controller Ready:- If the second LED is “ON” then the controller is ready. If this controller is “OFF” then the controller is not ready.

Cache Dirty:- If this LED is “ON” it means that there is still information stored in the cache. If this LED is “OFF” then the cache is empty.

Partner Fail:- If the LED on the left is “ON” controller B has failed. If the LED on the right is “ON”, controller A has failed.

Drive Busy:- If this LED is “ON” there is activity on the drive channels. If this LED is “OFF” then there is no activity on the drive channels.

Host Busy:- If this LED is “ON” there is activity on the host channels. If this LED is “OFF” then there is no activity on the host channels.

### Maintenance Modules

The maintenance modules are found in slots 8 and 9. When the mouse cursor is dragged over the maintenance module the following message appears.

IP:- The IP address shown in the message tag reflects the IP address that was assigned to the controller when RAIDWatch was being setup.

Mask:- The 255.255.255.0 subnet mask is used to distinguish the IP address.

Gateway:- This specifies the gateway IP address.

The maintenance modules come with both a green and a yellow LED. The green LED shows the Ethernet Link status and the yellow one shows the Data Transfer status.

Ethernet Link Status (green LED):- If this LED is “ON” the Ethernet link is functioning. If it is “OFF”, the Ethernet connection has not been established.

Data Transfer Status (yellow LED):- “Flashing” indicates that data is being transferred. “OFF” indicates that no data is currently being transferred.

### BBU Module

The BBU modules are found in slots 10 and 11. When the mouse cursor is dragged over the BBU module a message tag revealing the status of the BBU will appear.

The above message tag shows that the battery unit is fully charged. Other message tags include:

- "Battery not present"

- "Battery malfunctioning"

- "Battery in charging"

The BBU Module comes with a single, yellow LED that shows the status of the BBU. If the LED is “ON” either a battery has not been installed or the battery is bad. If the LED is “OFF”, the BBU is capable of sustaining the memory in the event of a power loss. If the LED is “Flashing” the BBU is charging.

### PSU Status Message

If you run the cursor over a PSU that is functioning normally, the following message will appear on the screen.

A missing or malfunctioning PSU will be easy to detect using the GUI. The PSU in the graphic that corresponds to the failed or missing controller head PSU will have a pulsing, red background. To find out what is wrong with the PSU, use the mouse to drag the cursor over the PSU in the GUI and a message tag will appear (see the figure below).

If the PSU is missing, the same message as shown in the figure above will appear when the cursor is dragged over the PSU. If the PSU is malfunctioning then the message will read “Power supply malfunctioning.”

### Cooling FAN Module Status

If you run the cursor over an individual that is functioning normally, the following message will appear on the screen.

In the above message the normal functioning of the fan is explicitly stated and in the parentheses next to this message the rotation speed of the fan is given.

A missing or malfunctioning cooling FAN will be easy to detect using the GUI. The cooling FAN in the graphic that corresponds to the failed or missing controller head cooling FAN will have a pulsing, red background. To find out what is wrong with the PSU, use the mouse to drag the cursor over the PSU in the GUI and a message tag will appear (see the figure below).

If the fan is malfunctioning the same message as shown in the figure above will appear when the cursor is dragged over the PSU. If the fan is missing then the message will read “Fan is not present.”

# B Java Runtime Environment

## Updated Version:

The CD that comes with your RAID appliance contains the JavaTM 2 JRE version 1.2.2. JavaTM 2 JRE. This JRE  is constantly updated and improvements are constantly being made. The latest available version can be downloaded from the Sun Microsystems web site.

## Updated Version:

According to the on-line documentation provided by Sun Microsystems,

- Java 2 updates have been made to address key customer issues and feedback.

- These updates DO NOT include any new features or functionality.

- The latest Java 2 update (version 1.2.2_014) has several key bug fixes.

- Detailed information on the latest Java 2 update can be found in the Release Notes of the update. The Release Notes can be found at: http://java.sun.com/products/jdk/1.2/ReleaseNotes.html.

- Sun Microsystems strongly recommends that users upgrade their systems to the latest Java 2 version.

- If a user decides to install the latest Java 2 version then the user should follow his/her standard test procedures and Java deployment to ensure that the system runs smoothly.

## Download and Install

To download and install the latest Java 2 version, please follow these instructions.

- The latest Java 2 version can be downloaded from http://java.sun.com/products/jdk/1.2/download.html.

- Once you have opened this web page, select the version that is compatible with your operating system (OS).

- After the latest Java 2 version has been downloaded from the Sun Microsystems’ web site the software must be installed.

- Installation instructions for the latest Java 2 version can be found at: http://java.sun.com/products/jdk/1.2/install.html.

- Please read through these instructions prior to installing the Java 2 JRE.

## Note:

- Latest Version:- At the time of writing this appendix the latest Java 2 version is version 1.2.2_014. Java 2 is updated regularly so please periodically check the Sun Microsystems web site to check for the latest Java 2 version.

- Web Address:- Sun Microsystems’ web address some times changes every few weeks/months. If the URLs given in this addendum are out of date, please use “Java 2 JRE version 1.2” as the key words in a search engine to locate the correct web site.

# C     Command Summary

This appendix describes the commands available in RAIDWatch Manager.  These commands are presented either in menus or as command buttons on the toolbar.

## Menu Commands

This section lists and explains the commands available from the menus in the menu bar.  Keyboard strokes for commands that can also be executed from the keyboard are indicated by angle brackets.  For example:

Connect	<C>

means that you can connect to a disk array system by selecting Connect from the File menu or by pressing the “C” key on your keyboard.

### RAIDWatch File Menu Commands

| Command | Description |
| --- | --- |
| Connect <C> | Connects RAIDWatch Manager to a particular disk array system for management. This command has the same function as the Connect command button on the toolbar. |
| Disconnect <D> | Disconnects from the current disk array system, terminating the current management session with the device but not exiting RAIDWatch Manager. |
| Agent <A> | Activates the Connect to RAID Agent dialog box. |
| Exit <X> | Closes the RAIDWatch Manager application. |

### RAIDWatch Open Menu Commands

| Command | Description |  |
| --- | --- | --- |
| Enclosure <E> | Displays the Enclosure window. This command has the same function as the Enclosure command button on the toolbar. |  |
| RAID View <R> | Displays the main RAID View window. This command has the same function as the RAID View command button on the toolbar. |  |
| Event Log <L> | Displays the Event Log window. This command has the same function as the Event Log and Event Flag command buttons on the toolbar. |  |
| Statistics <S> | Accesses the two statistics display commands (Cache Hits and Disk R/W). This command has the same function as the Statistics command button on the toolbar. |  |
|  | Cache Hits <C> | Displays the Cache Hits statistics window. This command has the same function as the Cache Hits button on the toolbar. |
|  | Disk R/W <D> | Displays the Disk Read/Write statistics window. This command has the same function as the Disk R/W button on the toolbar. |

### RAIDWatch View Menu Commands

| Command | Description |
| --- | --- |
| Tile In-Window <W> | This command arranges currently open windows so that they are all visible and occupy an equal part of the RAIDWatch application window. It has the same function as the Tile In-Window command button on the toolbar. |
| Tile In-Sequence <S> | (This command is disabled in the current version of the software.) |

### RAIDWatch Help Menu Commands

| Command | Description |
| --- | --- |
| Contents <C> | Displays RAIDWatch Manager on-line help. |
| About <A> | Displays information about the RAIDWatch Manager program. |

.

## Before You Begin

Before RAIDWatch’s event notification can be used, some adjustments may need to be made to your OS’s configuration.  See 2.3 Platform Requirements for information about pre-installation configuration needs.  Many of the steps described in section 2.3 directly relate to event notification, particularly for Windows OSes.

### Event Monitor Connect Menu Commands

| Command | Description |
| --- | --- |
| Open <O> | Connects Event Monitor to a particular disk array system. |
| Close <C> | Disconnects Event Monitor from a particular disk array system. |
| RAIDWatch <R> | When a disk array IP address is selected, this command opens the RAIDWatch Manager application. |
| Exit <E> | Closes the Event Monitor application. |

### Event Monitor About Menu Commands

| Command | Description |
| --- | --- |
| Help <H> | Displays on-line help. |
| About <A> | Displays information about the Event Monitor program. |

## Command Buttons

This section describes the various command buttons provided  to facilitate execution of commonly used commands.  These buttons are on the toolbar.

| Command Button | Description |
| --- | --- |
| Connect | Connects RAIDWatch Manager to a particular disk array system for management. This command has the same function as the Connect command in the File menu. |
| Disconnect | Disconnects from the current disk array system, terminating the current management session with the device but not exiting RAIDWatch Manager. This command has the same function as the Disconnect command in the File menu. |
| Enclosure | Displays the Enclosure window. This button has the same function as the Enclosure command in the Open menu. |
| RAID View | Displays the main RAID View window. This button has the same function as the RAID View command in the Open menu. |
| Event Log | Displays the Event Log window. This button has the same function as the Event Log command found in the Open menu. |
| Statistics | Displays the Statistics window options. Select either Cache Hits or Disk R/W. Has the same function as the Statistics command in the Open menu. |
| Tile | This command arranges currently open windows so that they are all visible and occupy an equal part of the RAIDWatch application window. It has the same function as the Tile In-Window command in the View menu. |

# D     Troubleshooting

This appendix provides troubleshooting tips for common problems you may encounter while using RAIDWatch Manager.  It includes the following topics:

- Common oversights

- Error codes

- Error messages

## D.1   Common Oversights

Check the following first before you run RAIDWatch or if you are having trouble.

### Check Platform Requirements

Review section 2.3 Platform Requirements to make sure that all OS-specific installation configuration steps have been completed.

### Check RAIDWatch Connections

In-Band SCSI (Solaris only) : All Solaris operating systems must have an in-band SCSI driver installed.

Agents : All RAID host servers must have a Secondary Agent installed and running.  Check that all RAID hosts have a Secondary Agent.

Windows NT Systems : Open the Control Panel, then open the Service icon in the Control Panel and check the status of “Primary Agent” and “Secondary Agent” to see if the status reads “Start.”

Unix Systems (including Linux) : Make sure the in-band SCSI driver is installed (Solaris only).  (You can run Text RAID Manager to determine this.)

- Check the Primary and Secondary agents

#ps -eaf|grep primary

#ps -eaf |grep secondary

If either program is not running, please open  a terminal session and run it manually.

IP Settings : Each RAID host server must have a unique IP address assigned.  Check the IP address of each RAID host.  Also, each Secondary agent IP and TCP port setting must be listed under the Primary agent’s configuration.  To check this, run the RAIDWatch install program at the Primary agent host and choose Configuration Only.

TCP Port Settings : The Primary agent must have a TCP port setting different from the Secondary agents.  The Secondary agents should have a common TCP port setting, and that setting should be listed with each Secondary agent’s IP under the Primary agent configuration.

### NPC Troubleshooting

(Windows NT servers only)

Fax : If NPC has trouble sending fax messages, please check to be sure that Microsoft Personal Fax is installed.

Broadcast : If NPC does not broadcast normally, please check to be sure that Windows Messaging is installed on BOTH server and client(s).

(RedHat)

rwalld: Under some circumstances, software component necessary for running NPC under RedHat - "rpc.rwalld" - may be absent from system.

If the package "rpc.rwalld" is not installed, users can find the associated rpms, "rwall-server-<version>-.i386.rpm" and "rwall-<version>.i386.rpm" under the directory "RedHat/RPMS" on the RedHat installation CD.

To install the rpm packages, use the command:

rpm -i <your rpm package>+.

## D.2   Error Codes

Under some circumstances, you may see the following error codes.  (Some additional error codes are not listed because they indicate failures which are not user resolvable.)

### Code: RC=1

Meaning : The Primary agent did not respond to a RAIDWatch Manager command before timeout.

What You Can Do : Check your network environment to determine whether or not the Primary agent is available. Use a “ping” utility to see if the server exists on the network.

### Code: RC=2

Meaning : The Primary agent did not understand the last  RAIDWatch Manager command.

What You Can Do : Please check the firmware version of controller to see if it is compatible with your version of RAIDWatch.  RAIDWatch currently only works with firmware versions 2.23 or 3.11 and higher.  You should also contact your technical support engineer.

### Code: RC=3

- Meaning : Exception error detected

What You Can Do : Check that the versions of RAIDWatch Manager, Primary agent, and Secondary agent are compatible.  You should also contact your technical support engineer.

### Code: RC=7

Meaning : RAIDWatch didn’t find the RAID controller.

What You Can Do : Check if the Secondary and Primary are running on their respective servers; also check if the RAID controller that RAIDWatch wants to control has failed.  (The Secondary agent on the host might be functioning, but the RAID controller it relates to has failed.)

### Code: RC=9

Meaning : The Primary or Secondary agent is not running, or one or more servers is down.

What You Can Do : Restart the suspect servers and all related RAIDWatch agents.

## D.3   Error Messages

You may encounter some of the following RAIDWatch Manager error messages.  This section provides some guidance for those messages that may be unclear.  Note that this is not an exhaustive list of RAIDWatch Manager error messages.

### Under Channel Settings

- Message : Undefined Channel Mode

Explanation : Channels must be defined as either a drive or host channel.

- Message : Drive channel should have at least one PID!

Explanation :  Please assign one PID to each drive channel.

- Message : Channel settings failed!

Explanation :  Please check the channel settings.

Message : Drive channel can only have one ID! Please remove ID from PID list first, and then add.

Explanation : Please remove the current ID from the PID list first, then add the new one.

### Under Logical Drive Creation

Message : Maximum RAID 3 or RAID 5 drive count is 31!

Explanation : You cannot create a RAID3 or RAID5 logical drive with more than 31 SCSI drives.  Reduce the number of drives you are using.

### Under Logical Drive Settings

- Message : Delete Logical Drive failed!

What can be done : The logical drive maybe a member of a LV, so it can not be deleted.

- Message : Rebuild failed!

What can be done : Use another new drive and try to perform the rebuild again.

- Message : Expand logical drive failed!

What can be done : Replace the hard drive and rebuild the logical drive.

- Message : Add drives failed!

Adding drive is a re-initialization process.  A failure during the process will be fatal.

What can be done : Delete the logical drive then recreate the logical drive.

- Message : Change LD assignment failed!

What can be done : Check if drive channel is assigned both PID and SID that own the member SCSI drive of this LD.

### Under RAIDView

- Message : Currently, controller supports 8 LDs.

Explanation : Currently, the controller can only support 8 logical drives.

- Message : Setup redundant controller failed!

Explanation : Check the redundant controller configuration.  See if the PID and SID of drive and host channels are all correct and if the logical drives (volumes) are correctly assigned.

All other RAIDWatch Manager error messages should be fairly self-explanatory.

# E     Glossary

Fault Bus     A proprietary enclosure management interface.  It is intended to be used to warn RAID managers of enclosure environment component failures.

Fibre    (Also known as “fibre channel.”)  A device (in the case of RAID, a data storage device) protocol capable of high data transfer rates.  Fibre channel simplifies data bus sharing and supports not only greater speed, but also more devices on the same bus.  Fibre channel can be used over both copper wire and optical cable.

Fiber     An optical network data transmission cable type which is unrelated to fibre channel (above).

HBA     Host-Bus Adapter – an HBA is a device that permits a PC bus to pass data to and receive data from a storage bus (such as SCSI or fibre channel).

Host     A computer, typically a server, which uses a RAID system (internal or external) for data storage.

Host LUN     (See Host and LUN).  “Host LUN” is another term for a LUN.

I2C     Inter-IC – a type of bus designed by Philips Semiconductors which is used to connect integrated circuits.  I2C is a multi-master bus, which means that multiple chips can be connected to the same bus and each one can act as a master by initiating a data transfer.

In-Band SCSI     (sometimes “in-band” or “In-band”) A means whereby RAID management software can use SCSI cabling and protocols to manage a controller.  (Note: in-band SCSI is typically used in place of RS-232 for controller management.)

ISEMS     Infortrend Simple Enclosure Management System – an I2C-based enclosure monitoring standard developed by Infortrend Technologies, Inc.

JBOD     Just a Bunch of Drives – non-RAID use of multiple hard disks for data storage.

JRE     Java Runtime Environment – the Solaris Java program used to run .JAR applications locally or over a network or the internet.

Logical Drive     Typically, a group of hard disks logically combined to form a single large storage unit.  More broadly, the assignment of a SCSI or Fibre channel ID to a drive or drives for use in storage management.  Often abbreviated, “LD.”

Logical Volume     A group of logical drives logically combined to form a single large storage unit.  Often abbreviated, “LV.”

LUN     Logical Unit Number – A 3-bit identifier used on a SCSI bus to distinguish between up to eight devices (logical units) with the same SCSI ID.

Mapping     The assignment of a protocol or logical ID to a device for purposes of data storage, data transfer, or device management.

Mirroring     A form of RAID where two or more identical copies of data are kept on separate disks.  Used in RAID 1.

NAS     Network Attached Storage – a RAID enclosure that includes a network interface so that the disk array can be directly connected to a LAN.

NPC     Notification Processing Center –  a software application included with RAIDWatch which permits event notification via various methods including e-mail and fax.

##### NRAID     Non RAID

Parity     Parity checking is used to detect errors in binary-coded data.  The fact that all numbers have parity is commonly used in data communications to ensure the validity of data. This is called parity checking.

Primary Agent     The RAIDWatch module which manages secondary agents and supports both NPC and RAIDWatch Manager.

Top of Form 1

RAID     Redundant Arrays of Independent Disks (Originally “Redundant Arrays of Inexpensive Disks”). The use of two or more disk drives instead of one disk, which provides better disk performance, error recovery, and fault tolerance, and includes interleaved storage techniques and mirroring of important data.  See Appendix D.

RAIDWatch Manager     The GUI RAID interface part of RAIDWatch.

SAF-TE     SCSI Accessed Fault-Tolerant Enclosures – an evolving enclosure monitoring device type used as a simple real-time check on the go/no-go status of enclosure UPS, fans, and other items.

SAN     Storage Area Network – is a high-speed subnetwork of shared storage devices.  A storage device is a machine that contains nothing but a disk or disks for storing data.  A SAN's architecture works in a way that makes all storage devices available to all servers on a LAN or WAN.  Because stored data does not reside directly on the network’s servers, server power is utilized for applications rather than for data passing.

SCSI     Small Computer Systems Interface (pronounced “scuzzy”) – a high-speed interface for mass storage that can connect computer devices such as hard drives, CD-ROM drives, floppy drives, and tape drives.  SCSI can connect up to sixteen devices.

Secondary Agent     The RAIDWatch module which manages and monitors a RAID controller and receives RAIDWatch Manager commands via the primary agent.

S.E.S.		       SCSI Enclosure Services is a protocol that is used to manage and sense the state of the power supplies, cooling devices, temperature sensors, individual drives, and other non-SCSI elements installed in a fibre channel JBOD enclosure.

S.M.A.R.T.     Self-Monitoring, Analysis and Reporting Technology – an open standard for developing disk drives and software systems that automatically monitor a disk drive’s health and report potential problems. Ideally, this should allow users to take proactive actions to prevent impending disk crashes.

Spare     (Local / Global) A drive designation used in RAID systems for drives that are not used but are instead “hot-ready” and used to automatically replace a failed drive.  RAIDs generally support two types of spare, Local and Global.  Local spares only replace drives that fail in the same logical drive.  Global spares replace any drive in the RAID that fails.

Stripe     A contiguous region of disk space. Stripes may be as small as one sector or may be composed of many contiguous sectors.

Striping  Also called RAID-0. A method of distributing data evenly across all drives in an array by concatenating interleaved stripes from each drive.

Stripe Size     (A.k.a., “chunk size.”) The smallest block of data read from or written to a physical drive. Modern hardware implementations let users to tune this block to the typical access patterns of the most common system applications.

Stripe Width     The number of physical drives used for a stripe. As a rule, the wider the stripe, the better the performance.

Write-back Cache     Many modern disk controllers have several megabytes of cache on board. Onboard cache gives the controller greater freedom in scheduling reads and writes to disks attached to the controller. In write-back mode, the controller reports a write operation as complete as soon as the data is in the cache. This sequence improves write performance at the expense of reliability. Power failures or system crashes can result in lost data in the cache, possibly corrupting the file system.

Write-through Cache     The opposite of write-back. When running in a write-through mode, the controller will not report a write as complete until it is written to the disk drives. This sequence reduces read/write performance by forcing the controller to suspend an operation while it satisfies the write request.

# F     RAID Levels

This appendix provides a functional description of Redundant Array of Independent Disks (RAID).  This includes information about RAID and available RAID levels.

## RAID Description

Redundant Array of Independent Disks (RAID) is a storage technology used to improve the processing capability of storage systems.  This technology is designed to provide reliability in disk array systems and to take advantage of the performance gains multiple disks can offer.

RAID comes with a redundancy feature that ensures fault-tolerant, uninterrupted disk storage operations.  In the event of a disk failure, disk access will still continue normally with the failure transparent to the host system.

RAID has six levels:  RAID 0 ~ 5.  RAID levels 1, 3 and 5 are the most commonly used levels, while RAID levels 2 and 4 are rarely implemented.  The following sections described in detail each of the commonly used RAID levels.

## Non-RAID Storage

One common option for expanding disk storage capacity is simply to install multiple disk drives into the system and then combine them end to end.  This method is called disk spanning.

In disk spanning, the total disk capacity is equivalent to the sum of the capacities of all SCSI drives in the combination.  This combination appears to the system as a single logical drive.  Thus, combining four 1GB SCSI drives in this way, for example, would create a single logical drive with a total disk capacity of 4GB.

Disk spanning is considered non-RAID due to the fact that it provides neither redundancy nor improved performance.  Disk spanning is inexpensive, flexible, and easy to implement; however, it does not improve the performance of the drives and any single disk failure will result in total data loss.

## RAID 0

RAID 0 implements block striping where data is broken into logical blocks and striped across several drives.  Although called RAID 0, this is not a true implementation of RAID because there is no facility for redundancy.  In the event of a disk failure, data is lost.

In block striping, the total disk capacity is equivalent to the sum of the capacities of all SCSI drives in the array.  This combination of drives appears to the system as a single logical drive.

RAID 0 provides the highest performance without redundancy.  It is fast because data can be simultaneously transferred to/from multiple disks.  Furthermore, read/writes to different drives can be processed concurrently.

## RAID 1

RAID 1 implements disk mirroring where a copy of the same data is recorded onto two sets of striped drives.  By keeping two copies of data on separate disks or arrays, data is protected against a disk failure.  If, at any time, a disk on either side fails, the good disks can provide all of the data needed, thus preventing downtime.

In disk mirroring, the total disk capacity is equivalent to half the sum of the capacities of all SCSI drives in the combination.  Thus, combining four 1GB SCSI drives, for example, would create a single logical drive with a total disk capacity of 2GB.  This combination of drives appears to the system as a single logical drive.

RAID 1 is simple and easy to implement; however, it is more expensive as it doubles the investment required for a non-redundant disk array implementation.

In addition to the data protection RAID 1 provides, this RAID level also improves performance.  In cases where multiple concurrent I/Os are occurring, these I/Os can be distributed between two disk copies, thus reducing total effective data access time.

## RAID 1(0+1)

RAID 1(0+1) combines RAID 0 and RAID 1 – Mirroring and Disk Striping. RAID (0+1) allows multiple drive failure because of the full redundancy of the hard disk drives. If more than two hard disk drives are chosen for RAID 1, RAID (0+1) will be performed automatically.

- IMPORTANT:

RAID (0+1) will not appear in the list of RAID levels supported by the controller. If you wish to perform RAID 1, the controller will determine whether to perform RAID 1 or RAID (0+1). This will depend on the drive number that has been selected for the logical drive.

## RAID 3

RAID 3 implements block striping with dedicated parity.  This RAID level breaks data into logical blocks, the size of a SCSI disk block, and then stripes these blocks across several drives.  One drive is dedicated to parity.  In the event a disk fails, the original data can be reconstructed from the parity information.

In RAID 3, the total disk capacity is equivalent to the sum of the capacities of all SCSI drives in the combination, excluding the parity drive.  Thus, combining four 1GB SCSI drives, for example, would create a single logical drive with a total disk capacity of 3GB.  This combination appears to the system as a single logical drive.

RAID 3 provides increased data transfer rates when data is being accessed in large chunks or sequentially.

However, in write operations that do not span multiple drives, performance is reduced since the information stored in the parity drive needs to be re-calculated and re-written every time new data is written to any of the data disks.

## RAID 5

RAID 5 implements multiple-block striping with distributed parity.  This RAID level offers the same redundancy available in RAID 3; though the parity information this time is distributed across all disks in the array.  Data and relative parity are never stored on the same disk.  In the event a disk fails, original data can be reconstructed using the available parity information.

For small I/Os, as few as one disk may be activated for improved access speed.

RAID 5 offers both increased data transfer rates when data is being accessed in large chunks or sequentially and reduced total effective data access time for multiple concurrent I/O’s that do not span multiple drives.

# G     Additional References

This appendix provides direction to additional references that may be useful in creating and operating a RAID, and in using RAIDWatch and RAIDWatch Manager.

## Java Runtime Environment

JRE (Java Runtime Environment) is a shareware product from Sun/Solaris.  Two websites that may be of use relative to JRE are:

The main Java website URL:

java.sun.com

The JRE download website URL:

www.sun.com/software/solaris/jre/download.html

## RAIDWatch Update Downloads & Upgrading

Infortrend will provide RAIDWatch agent and RAIDWatch Manager updates periodically both via our ftp server and as new CD releases.  Our FTP site can be accessed via our websites at:

www.infortrend.com

www.infortrend.com.tw

## Uninstalling RAIDWatch

RAIDWatch agents and RAIDWatch Manager can be uninstalled.  Choose the Uninstall icon in the RAIDWatch group or type “uninstall” in the usr/hybrid/bin subdirectory under Unix-based systems.

B

block striping with dedicated parity	B-32

C

command buttons	B-11, B-14

controller head	A-1

D

dedicated parity	E-5

disk mirroring	E-30

disk spanning	E-28

E

Enclosure command button	  B-12

ER2510FS	A-1

M

menu bar	B-11

P

Panel View	A-2, A-3

R

RAID 0	E-28, E-29, E-31

RAID 1	E-30, E-31

RAID 1(0+1)	E-31

RAID 3	E-32, E-33

RAID 5	E-33

RAID levels	E-28, E-31, F-35

rear view	3

T

troubleshooting	C-16

A     ER2510FS RAID Controller Head	1

Connecting to the RAID Agent	1

The Panel View	2

Accessing the Panel View	3

Viewing System Information	4

B     Command Summary	11

Menu Commands	11

Before You Begin	13

Event Monitor Connect Menu Commands	14

Event Monitor About Menu Commands	14

Command Buttons	14

C     Troubleshooting	16

C.1   Common Oversights	17

C.2   Error Codes	19

C.3   Error Messages	20

D     Glossary	23

E     RAID Levels	29

RAID Description	29

Non-RAID Storage	29

RAID 0	30

RAID 1	31

RAID 1(0+1)	32

RAID 3	33

RAID 5	34

F     Additional References	36

Java Runtime Environment	36

RAIDWatch Update Downloads & Upgrading	36

Uninstalling RAIDWatch	36