# 1     Introduction

- This chapter provides information about the RAIDWatch management program, including the following topics:

- Product description

- Feature summary

- Feature highlights

- Conceptual foundation

## 1.1     Product Description

Infortrend’s GUI RAID Manager, “RAIDWatch,” is a Java-based program specifically designed for use in managing disk array systems implemented using any of Infortrend’s line of standalone RAID controllers (GUI stands for “graphic user interface.”)  RAIDWatch provides a user-friendly interface that facilitates understanding of the relationship between disk array elements and simplifies the normally complicated process of array configuration.  RAIDWatch also provides real-time reporting on the status of the entire array, thus making the task of monitoring disk arrays virtually effortless.  Since the release of software revision 1.31, Event Monitor is supplemented for use with a constant monitoring of multiple disk arrays.

RAIDWatch complements the on-board console interface found on Infortrend’s non-PCI RAID controllers and a line of host-based, text-based RAID Managers providing the same functionality, but with greater ease of use.  The following sections describe the outstanding features of RAIDWatch and introduce its conceptual framework.

## 1.2     Feature Summary

The list below summarizes RAIDWatch features.

- User-friendly graphical interface running under operating systems compatible with the Java Run-time Environment

- Internet browser access to full program functionality provides worldwide management capability

- Supports Infortrend’s entire line of Fibre-to-Fibre, Fibre-to-SCSI, and SCSI-to-SCSI RAID controllers

- Communicates with the controllers over a LAN, the Internet, over the SCSI bus or Fibre channel

- Supports multiple instances of RAID managers over the network, allowing multiple management sessions with a disk array system

- Illustrates graphically and clearly the relationship between various disk array elements

- At a glance monitoring of the entire disk array status by RAIDWatch and constant monitoring of multiple systems by Event Monitor

- Supports remote management over the network of an agent running Windows NT, Solaris, or Linux via the TCP/IP protocol (future versions will support additional protocols)

- Provides standard disk array functions, including examining and modifying controller configuration; viewing and monitoring configuration and status of physical drives; scanning in new physical drives; creating, deleting, and monitoring configuration and status of logical drives; rebuilding logical drives; defining spare drives; creating, deleting, and partitioning logical volumes; and mapping logical drive and volume partitions to specific host channels/LUNs

- Enclosure management functions, including defining multiple customizable enclosures, dimensions, and number of drives; monitoring physical drive, power supply, fan, and temperature status; displaying the relative location of failed physical drives for reduced risk of replacing the wrong drives

- Supports redundant configuration of important RAIDWatch modules to avoid single-point-of-failure

- RAID controller realtime event notices provide information about various event occurrences, including the time when an event occurs, event severity, and event description.

- Selectable event notification via SNMP traps by severity levels

- Supports statistics monitoring for displaying I/O throughput and cache hit rate

- Provides innovative, user-configurable event notification functions

- Pager notification via a local modem

- E-Mail notification via the MAPI service of Windows NT or Windows 95; or built-in SMTP for Unix systems

- Broadcast notification over the LAN:

- Broadcasts user-configurable message along with the event description (Broadcast notification currently not supported on cross-OS, e.g., Unix to Windows, platforms.)

- Facsimile (Fax) notification via a local Fax/modem:

- User-configurable fax messages sent along with the event description

- Automatic message retransmission in the event previous transmission attempts fail

- Provides password protection for guarding against unauthorized modification of disk array configuration.

## 1.3     Feature Highlights

- This section explains in greater detail the important features of RAIDWatch.

### 1.3.1	Graphical User Interface

- RAIDWatch’s graphical interface is designed for ease-of-use.  It uses symbolic icons to represent physical and logical drives, and logical volumes on the screen; and to identify the current configuration of a disk array system.  Pull-down and pop-up menus are used with all command options listed.

Users need only point and click a mouse button to select an icon or command.  The program also identifies the current status of various drives by changing the colors of their respective icons.

With an easy-to-use interface, complicated disk array operations such as logical drive and logical volume creation, drive partitioning, and drive partition mapping to host channels/LUNs can be completed with only a few mouse clicks.

### 1.3.2	Enclosure Management

The enclosure window provides real-time reporting of the status of the connected physical drives.  When a drive fails, the system highlights the corresponding icon of the failed drive by placing a red X mark on it; when you remove a drive, its icon is removed from the enclosure window.  This feature is particularly useful in cases where a drive fails, and you need to identify its exact location for subsequent replacement.

The enclosure window also includes Logical view as a sub-function.  The Logical view displays the logical relationship among member drives of a logical configuration.  Drives belonging to the same logical unit will be displayed in the same color for ease of identification.

### 1.3.3	Powerful Event Notification Function

- RAIDWatch can notify system administrators of event occurrences and status changes in the disk array system.  Notifications can be sent via a modem to a pager, via the Internet as E-mail messages, via a LAN as a broadcast message, SNMP traps, or via fax/modem as fax messages.

### 1.3.4	Java-based Remote Management

- RAIDWatch supports remote management of Infortrend’s disk array controllers over a LAN/WAN or the Internet using the TCP/IP protocol.  Management over the LAN is achieved through data exchanges between a remote RAIDWatch Manager station and RAIDWatch agents on the host server(s).  The following figure shows a typical connection:

### 1.3.5	Password Protection

- RAIDWatch Manager comes with password protection to prevent unauthorized users from modifying the configuration of the disk array system.  With the password security feature, you have the luxury of leaving your RAIDWatch station unattended knowing that the currently managed disk array system is safe from any unauthorized modifications because the correct password must be entered for each modification.

## 1.4     Conceptual Foundation

- In order for RAIDWatch to function properly, different software modules must be correctly installed on different servers and stations in a given LAN or WAN.  Assuming that a given network has multiple RAID systems, one RAID connected server will need to be chosen as the main server.  This point is particularly important if RAIDWatch will be operated via web browsers as the main server will need to be a web server.

The main server will need to have the Primary agent, Secondary agent (if the main server is also a RAID host), and Notification Processing Center (NPC) if it will be used.  The main server will also need Java Run-time Environment (JRE) installed if it is a RAIDWatch Manager or Event Monitor site.  Subsequent installations at other RAID servers will only need the Secondary agent installed.

The table below provides a guide to what modules need to be installed where (items in the bottom five rows of the table are not included with RAIDWatch and must be installed or modified by system users):

For more information about specific platform requirements, see Section 2.3 Platform Requirements.

Table 1-1  :  RAIDWatch Module Requirements

| Install Elements | Main RAID Server* | Web Server* | Secondary Server | RAIDWatch Station |
| --- | --- | --- | --- | --- |
| Primary Agent |  |  |  |  |
| Secondary Agent |  |  |  |  |
| RAIDWatch Manager |  |  |  |  |
| NPC |  |  |  |  |
| JRE |  |  |  |  |
| OS drivers1 |  |  |  |  |
| OS patches2 |  |  |  |  |
| Web browser |  |  |  |  |
| Edit browser preferences3 |  |  |  |  |
| Web server |  |  |  |  |

* If RAIDWatch is going to be operated or installed through web browsers, the web server

must be the main RAID server.

1 OS drivers are required for Solaris servers.

2 OS patches are required to run JRE on Solaris, AIX, and Linux servers.

3 Browser preferences must be edited for browsers running under Windows OSes.

The requirements for installing Event Monitor are exactly the same as those for RAIDWatch Manager.