# 3     Basic Operations

This chapter discusses basic RAIDWatch Manager operations.  We recommend that you review it to learn the basic organization and functions of the program.  This chapter includes the following topics:

- Starting RAIDWatch Manager

- Connecting and disconnecting from a disk array

- Setting up security

- Display controls

- Exiting RAIDWatch Manager

## 3.1     Starting RAIDWatch

### 3.1.1	Starting RAIDWatch Agents and NPC

Under Windows (NT 4, or 2000) OSes:

The Primary Agent and Secondary Agents start automatically under Windows OSes each time the host computer is reset.  However, the NPC must be started manually by executing the NPC.exe each time the host is reset.

Under Unix or AIX OSes (Solaris 7 SPARC or x86, HP UX 11, or Red Hat Linux 6.1):

Under Unix systems the Primary Agent, Secondary Agent, and NPC must be started manually each time the host computer is reset.

To start the Primary Agent, Secondary Agent(s), and NPC under a Unix system:

1.	At the host computer, change directories to:

/usr/hybrid/bin/   (or whatever directory you chose during the installation if not the default)

2.	Then at the command line, type:

primary <Enter>    to start the Primary Agent

secondary <Enter>    to start the Secondary Agent

npc <Enter>    to start the NPC

RAIDWatch is now running.  The next step is to start the GUI part of the software, RAIDWatch Manager.

### 3.1.2	Starting RAIDWatch Manager

The GUI management interface, RAIDWatch Manager, needs to be started by a network or RAID systems manager regardless of which OS is being used.

Depending on your setup, you can start RAIDWatch Manager in various ways.

For both local and distant management, and under various OSes, starting the program is fairly simple.  Please refer to the appropriate sub-section below for information.

Starting RAIDWatch Manager locally or via LAN under the Windows (95/98/Me/NT/2000) environment:

1.	From the Start menu, select Programs  RAIDWatch Manager.

-or-

Double-click the RAIDWatch Manager icon either in the group folder or from the desktop if a shortcut was added during the installation process.  The RAIDWatch Manager “Connect to RAID Agent” prompt window should appear on the screen.

2.	Enter the IP address and TCP port assignment of the disk array system where the Primary Agent was installed.  If you are running RAIDWatch Manager at the Primary Agent host machine (i.e., “locally”) and want to manage a RAID hosted by the Primary machine, click the Default button.

3.	Double click on a RAID host IP you would like to manage, then double click on the controller icon, then double click on the connection method (e.g., In-Band SCSI), to connect to the disk array system.  For more information on how to connect, see the Connecting and Disconnecting from a Disk Array section of this chapter.

Starting RAIDWatch Manager for remote management via web browser (any supported OS):

1.	Start your web browser and enter the IP address of the Primary Agent host followed by GRM.HTML as your URL (e.g., 222.212.121.123\GRM.HTML).  After a brief delay while the Java Applet starts, the RAIDWatch Manager main connection window appears on the screen.

2.	Double click on a RAID host IP you would like to manage, then double click on the controller icon, then double click on the connection method (e.g., In-Band SCSI), to connect to the disk array system.  For more information on how to connect, see the Connecting and Disconnecting from a Disk Array section of this chapter.

Starting RAIDWatch Manager locally or via a LAN under a Unix or AIX workstation (Solaris 7 (SPARC, x86); Red Hat Linux 6.1) environment:

1.	Open a terminal application or command line window.

2.	Change directory to /usr/hybrid/bin/ (or whatever directory you chose during the installation if not the default).

3.	At the command prompt, type:

java -jar grm.jar

The RAIDWatch Manager main connection window should appear on the screen.

4.	Enter the IP address and TCP port assignment of the disk array system where the Primary Agent was installed.  If you are running RAIDWatch Manager at the Primary Agent host machine (i.e., “locally”) and want to manage a RAID hosted by the Primary machine, click the Default button.

5.	Double click on a RAID host IP you would like to manage, then double click on the controller icon, then double click on the connection method (e.g., In-Band SCSI), to connect to the disk array system.  For more information on how to connect, see the Connecting and Disconnecting from a Disk Array section of this chapter.

## 3.2     Connecting and Disconnecting from a Disk Array

Before management can be performed on a particular disk array system, you need to first establish a connection between your RAIDWatch Manager station and the Primary Agent host.  Once a connection is established successfully, management can be started.

Disconnection is used for breaking the link between the RAIDWatch Manager station and the array.  This option is particularly useful in cases where multiple disk arrays are being managed at the same time  instead of restarting the RAIDWatch Manager every time you need to switch to another system, you just need to disconnect from the current array and then connect to another.

The following discusses how to connect to a disk array.  Information on disconnection is provided at the end of this section.

 Connecting to a RAID system while working from the local Primary Agent host:

1.	From the File menu, select Connect.

-or-

Click on the Connect command button.  The following prompt will appear on the screen:

Click the Default button.

- Select the IP address of the RAID you would like to monitor or manage from the Connection View list.  Double click the IP address.  Double click the controller icon.  Double click the connection method (e.g., In-Band SCSI).

3.	You will be prompted for a password to access the controller.  Enter the password (there is no default password) and click OK.  The connection is successful when the RAID View Introduction window appears and the tool bar buttons are activated.

Connecting to a RAID system from a distant host:

1.	Select the IP address of the RAID you would like to monitor or manage from the Connection View list.  Choose and double click on an IP address.  Double click the controller icon for that IP address.  Choose and double click the connection method (e.g., In-Band SCSI).

2.	You will be prompted for a password to access the controller.  Enter the password (there is no default password) and click OK.  The connection is successful when the Introduction window appears and the tool bar buttons are activated.

 Disconnecting from a disk array system:

- From the File menu, select Disconnect.  Or, click the Disconnect button on the toolbar.

All toolbar buttons, except the Connect command button, turn gray signifying disconnection from the disk array system.  RAIDWatch Manager should return to the Connection View window.

## 3.3     Setting Up Security

RAIDWatch provides password protection to prevent unauthorized access to RAID controllers or controller setting modifications.  This protection, which is implemented by the RAIDWatch Primary Agent, prompts a user for the station password the first time he or she attempts to connect to a controller through a Secondary Agent.

By default, RAIDWatch comes without passwords, so when prompted for a password the first time, just press Enter.  After gaining control, set a password to provide security to the managed disk arrays.

 Setting a password for RAIDWatch controller access:

1.	Display the Configuration window by clicking on the Configuration button under the introduction.

2.	From the Configuration window, click on the controller tab, under Controller, click on the operation tab, under Operation, choose the Password tab.  The Change Password dialog box appears:

3.	Type in a New Password.

4.	Re-type the password in the Verify Password field to confirm.

5.	Click OK.

 Setting TCP Port Numbers:

A security related setting that should be considered is the TCP port setting of Primary Agent hosts.  TCP port assignments must be made in order for RAIDWatch to work.  Modifying TCP port settings to something other than the default will increase system security.

RAIDWatch is shipped with a default TCP port setting for the Primary Agent: 58632.  It is recommended that users change the default TCP port setting to a less commonly used TCP port assignment.

TCP ports can be assigned any number between 1 and 65535, but as some of the smaller possible assignments are commonly used for other purposes, we recommend using a port number between 49152 and 65535.  Refer to Section 2.4.2, Installing RAIDWatch for information on configuring Primary Agent TCP port numbers.

## 3.4     Look and Feel

Because RAIDWatch Manager is a Java-based GUI program, it can accommodate the “Look and Feel” standards of various OSes.  At present, three different interface appearances are supported: Windows, Unix, and Java.

RAIDWatch Manager will auto-detect and configure to match the OS where it is currently running.

In the event of a compatibility problem or under unknown OSes or OS versions, the program will default to Java look and feel.

(Screen captures throughout this document show the Windows look and feel.)

## 3.5     Display Controls

RAIDWatch Manager provides a toolbar located just beneath the menu bar for displaying key command buttons.  These buttons serve various purposes, which are described in detail in this section.

Just like other GUI-based applications, RAIDWatch Manager works entirely with windows, buttons, and menus to facilitate various disk array operations.  These windows follow the standard Windows and Unix OS “Look and Feel” specifications, thus steps for manipulating elements and windows within any RAIDWatch Manager window generally conform to standard procedures.  The management sessions are best displayed in the 800x600 screen resolution.

### 3.5.1	Using the RAID View Window

After starting RAIDWatch Manager, choosing a RAID to manage, and entering the right password to get access, the RAIDWatch Manager main RAID View window appears on the screen.  All configuration operations pertaining to the disk array system will be performed within RAID View.

#### Basics

When RAID View first opens, the Navigation panel in the upper left quadrant of the window will display icons for the controller, Logical View, and Physical View.  The large Contents panel to the right will display an Introduction with access buttons for Configuration, Logical View, and Physical View.  Each of these primary function windows will be described below and in detail in Chapter 4.

The following describes the various control and display components found in the RAID View window:

- The menu bar displays the available menus.  All menus provide a list of commands for invoking various disk array and display-related operations.  Most commonly used commands such as Statistics and Event Log also have command buttons to facilitate their execution.  You can either select the command from the menu, or click on its toolbar command button.  For a summary of commands, see Appendix A, Command Summary.

- RAIDWatch Manager provides command buttons to execute most commonly used commands.  These buttons are displayed on the toolbar.  The following figure displays the command buttons on the toolbar:

- The Connect command button is used for connecting to a disk array system.  This button has the same function as the Connect command under the File menu.

- NOTE:

Multiple simultaneous RAIDWatch Manager connects to one Secondary agent is not a supported function.

- The Disconnect command button is used for disconnecting from a disk array system.  This button has the same function as the Disconnect command under the File menu.  (Note that Disconnect does not close RAIDWatch Manager.)

- The Enclosure command button displays the Enclosure window for displaying and configuring custom enclosures.  This button provides the same function as the Enclosure command under the Open menu.

- The RAID View command button displays the configuration and control window for the controller and drives.  This button provides the same function as the RAID View command under the Open menu.

- The Event Log command button opens the Event Log window for displaying the array event log.  This button provides the same function as the Event Log command under the Open menu.

- The Statistics command button displays the Statistics window for viewing activity (Cache hits or Read/Write) on the disk array system.  This button provides the same function as the Statistics command under the Open menu.

- The Tile command button arranges the displayed windows by giving each an equal share of the available application window space.  This button has the same function as the Tile In-Window command under the View menu.  (Note: The Tile In-Sequence function listed under View is not currently available.)

- The following additional commands are only accessible via the menu bar:

- The Agent command under the File menu brings up the “Connect to RAID agent” prompt.  This command is only available when RAIDWatch Manager is not currently connected to any agents.

- The Exit command under the File menu is always available and is used to end the current RAIDWatch Manager session.

- The Contents command under the Help menu brings up the main navigation window for the RAIDWatch Manager help file.

- The About command under the Help menu brings up a window that provides RAIDWatch version information.

- The windows display area is where the system displays RAIDWatch Manager windows.  You have the option to arrange the displayed windows on this area in various ways depending on your specific needs.  The View menu provides you with commands to arrange windows.  For information on how to do this, see the Arranging Windows section of this chapter.

- The RAID View window provides the configuration, installation, management, and monitoring functions available in RAIDWatch.  The RAID View window includes a Navigation Panel and a Content Panel.

- The Navigation Panel provides a tree organization display of logical and physical drives managed by the current controller.  The Navigation Panel also has a sub panel for displaying longer information trees.

- The Content Panel displays information about controller settings, logical drive settings, and physical drives as selected in the Navigation Panel.

- The status bar displays the results of various disk array operations.

- The scroll bars let you move parts of a window into view when the entire window does not fit into the windows display area.

### 3.5.2	Using the Configuration View

Configuration functions can only be accessed by clicking the Configuration button under the Introduction in the RAID View window.

Once in the Configuration view, all windows are accessed via window tabs.

Each tab corresponds with a controller configuration setting, option, or readout; and most tabs include various sub-functions.

See section 4.4   Configuring the Controller of this user’s manual and the controller’s user documentation for more information about controller settings.

### 3.5.3	Using the Logical View

Logical View is where you perform management on the logical drives and logical volumes of the disk array system.  Logical drives (LDs) are combinations of physical drives, which are used to create logical volumes (LVs).  These volumes (or their partitions) can then be mapped to various host LUNs.

A sub-function of the RAID View window, Logical View allows you to create, expand, and delete existing LDs and LVs.  It uses unique colors to distinguish between logical drives.  When a physical drive within a logical drive fails, the system notifies you by darkening the color of the affected logical drive.  The logical drive will remain in this state until either a spare is detected and an automatic rebuild is started, or the drive is replaced and a rebuild is manually initiated.

After a rebuild is complete, the logical drive will display its normal color, signifying an on-line condition.

To display the Logical View, you can either click on the Logical View button in the RAID View Introduction or select the Logical View icon in the navigation panel of the RAID View window.  A window similar to the one above will appear.

Note that the symbols in the Logical View initial content panel are not interactive.  To select a Logical Drive or Logical Volume, click on the appropriate icon in the navigation panel.

If you need information about a particular logical drive, just let the mouse pointer hover over its corresponding icon.  A readout similar to the following will appear:

This table displays the controller assignment, ID number, RAID level, total capacity, and current status of the logical drive.

For more information on how to create, delete, and rebuild logical drives and logical volumes, see Chapter 4, Array Management.

### 3.5.4	Using the Physical View

Physical View under the RAID View window is where you can view and modify the configuration of drive and host channels, the fault bus, and physical drives.

To display the Physical View, you can either click on the Physical View button in the RAID View Introduction or select the Physical View icon in the navigation panel of the RAID View window.  A window similar to the one below will appear.

Note that the symbols in the Physical View initial content panel are not interactive.  To select a channel, status display, or physical drive, click on the appropriate icon in the navigation panel.

Letting the mouse pointer hover over a channel displays a table similar to the following:

This table provides information such as primary and secondary IDs, and the current transfer clock rate.

The Physical View also allows you to modify the configuration of the drive and host channels, and scan in newly added or replaced physical drives.  It also provides host channel LUN configuration; and remote enclosure monitoring via I2C Bus, SAF-TE Bus, and Fault Bus.

To display host channel LUN configuration information, click on a host channel icon in the navigation panel, then click on the channel ID in the sub-navigation panel (lower-left quadrant of the window). For more information about modifying these configurations, see Chapter 4, Array Management.

#### I2C

In addition to displaying drive and host channels, the Physical View can also display I2C Bus Device Status. I2C is an interface by which signals from enclosure sensors are passed to the RAID controller.  It provides a basic status report on enclosure devices like power supplies, fans, and drive slots.

If your enclosure has I2C monitoring circuitry and cabling installed, RAIDWatch Manager will display an I2C icon under Physical View.  Click on the icon to show I2C devices and status.  I2C is not user configurable via RAIDWatch Manager.

#### SAF-TE

SAF-TE stands for SCSI Accessed Fault-Tolerant Enclosures. It is an enclosure management technology. A SAF-TE-compliant enclosure monitors the fan temperature, power supply, UPS, and also provides drive status LED’s.

The SAF-TE enclosure connects to the RAID Controller via a SCSI connector. The RAID controller communicates with the SAF-TE enclosure using standard SCSI commands.

#### How does SAF-TE work?

The SAF-TE device (often a back plane within a drive-bay enclosure) must occupy a connector on one of the drive channels’ SCSI cables. The presence of a SAF-TE device will be detected and its presence will be displayed in the RAIDWatch Manager program under Physical view.

SAF-TE controllers and SAF-TE devices are not user-configurable via RAIDWatch Manager.

#### Fault Bus

In addition to displaying drive and host channels, the Physical View can also display the status of the Fault Bus.

Fault Bus is a proprietary enclosure management interface. It will warn the user if a dangerous failure occurs within the RAID system. Fault Bus monitors the fault signals of the ventilation fans, power supply, enclosure temperature sensor, and UPS. In RAIDWatch Manager, it will report failures to the user by displaying a red “X” on the icon of whichever device has failed.

Fault Bus only detects failure signals; it does not detect the current temperature, fan rotation, power supply output, or UPS status.  Either a user-designed circuit or a third-party circuit is necessary for Fault Bus.

NOTE: Only the IFT-3101, IFT-3102, SentinelRAID 100, and SentinelRAID 150 Infortrend controllers support fault-bus.

The user is given the choice of enabling or disabling failure signals. In addition, each signal must be configured as either High or Low (refer to your third-party manufacturer’s documentation for the proper settings.)

### 3.5.5	Using the Enclosure Window

- The Enclosure window allows you to define enclosures for creating an exact replica of the disk array’s drive bay arrangement, displaying the exact location of the physical drives and controllers.

- The Enclosure window appears when you click on the Enclosure command button or select the Enclosure command from the Open menu. The command allows you to access the pre-configured enclosure(s). If multiple enclosures have been defined by system vendor, you may select your enclosure on the selection box.

- Select your enclosure and start adding devices into the empty spaces in the enclosure window.

Both enclosure windows allow you to assign locations for different components.  The Enclosure window is particularly useful in monitoring the status of the physical drives.  It provides you with a real-time report on the drive status, using symbols and colors to represent various conditions.  The following figures exemplify how RAIDWatch Manager represents various drive conditions:

Spare drives appear with their colors darker (shaded) than normal drives and have a red cross superimposed on them.

The red cross is larger on Global spares and smaller on Local spares.  When you remove a drive from the drive bay, its corresponding icon on the Enclosure window disappears.  The system places a large red X mark on the icon of a failed drive.

- NOTE:

Physical View under the RAID View window also provides a real-time report on drive status, using the same symbols and colors to represent various conditions.  What you see in the Enclosure window is also reflected in the Physical View.  These windows, however, differ in the way physical drives are presented; in the Enclosure window, the drives should be arranged according to their actual locations in the drive bays, while in the Physical View, the drives are arranged according to channel connections.

You can also display some information about a particular drive by simply placing the mouse pointer on its respective icon.  A readout similar to the one below appears.

This readout displays the current configuration of the drive, including the channel number of the connector on the controller to which the drive’s cable is connected, the ID number where the drive is installed, the drive’s capacity, transfer rate, and current status.

### 3.5.6	Using the Event Log Window

The Event Log window displays controller and array events since the last time RAIDWatch Manager was started.  The Event Log can be accessed through either the Event Log command button on the tool bar or under the Open menu.  If you need to read event logs recorded before RAIDWatch is started, consult your Event Monitor.

Event Log items include critical alerts, warnings, and notifications regarding the RAID controller; drives status; logical device status; and enclosure elements like power supplies, fan, and temperature.  Events are not always failures.  Some events, such as controller setting changes, are displayed for information purposes.

When events occur, RAIDWatch will display a waving Event Flag command button on the tool bar:

Clicking on the Event Flag button will open the Event Log window.  Event Log entries include the time of occurrence and a description of what event took place.

### 3.5.7	Using the Statistics Window

The Statistics window includes separate displays for cache hits and for sustained read/write performance.

Cache hits average and history provides information about the current operating performance of the RAID controller and disk array.

Cache Hits Average is a measure of data read or write cache accesses at the most recent moment of sampling.  It indicates what percentage of data I/O is cache accessed.

Cache Hits History shows cache read and write hit data over the last few minutes and indicates data caching consistency and frequency.

Sustained Read/Write Performance is also displayed as both an average and historically.  Read/write performance is another way to evaluate the current RAID controller and disk array I/O throughput.

Read/Write Average is a measure of the average data throughput, in MB/second, at the most recent data sample.

Read/Write History shows read/write performance over the last few minutes and indicates data throughput consistency.

The read/write data display scale can be modified using the Adjust button at the bottom-center of the Read/Write Statistics window.  Scales from 10MB/second to 200MB/second are available.

### 3.5.8	Arranging Windows

The View menu provides you with a command for rearranging the currently open RAIDWatch Manager windows.  You can manually manipulate the window frames to display them as you like, or use the Tile In-Window command under the View menu to arrange open windows to fit next to each other on the screen.  Tile In-Window is also available via a command button on the tool bar.

(Note: currently, the Tile In-Sequence function under the View menu is not supported.)

## 3.6     Exiting RAIDWatch Manager

Exiting from RAIDWatch Manager terminates the current management session with the disk array system.

 Exiting from RAIDWatch Manager:

- From the File menu, select Exit.

-or-

- Click the Close button on the program window.

Exiting from RAIDWatch Manager connected via web browser:

- From the File menu, select Exit.  (Recommended method.)

-or-

- Exit the browser application.

-or-

- Change the browser HTTP address to a URL or IP other than that of a RAIDWatch Primary Agent.