IEEE P802.11
Wireless LANs

Example of 5 Criteria: 1000BASE-T

1000BASE-T 5 Criteria

1. Broad Market Potential

· Broad set of applications

· Multiple vendors, multiple users

· Balanced cost, LAN vs. attached stations

-----------------------------------------------------------------

Faster CPUs, graphic  applications and general increases in network

traffic are forcing the  development of new LANs with higher

bandwidth. Most Ethernet  networks today incorporate nodes and

cabling capable of 100 Mb/s  operation. State-of-the-art network

servers today can generate  network loads of more than 400 Mb/s. A

1996 survey of 100  companies by Currid & Company found that

more than 40% of the  respondents indicated that they would need

100Mb/s-1000Mb/s  backbone solutions by the year 2000. Many

network backbones  already require bandwidth in excess of 100

Mb/s today; some  applications do as well.

Many applications and environments will benefit from this

capability:

· Building-level backbone, server and gateway connectivity

· Multimedia, distributed processing, imaging, medical, CAD/CAM

and pre-press applications

· Aggregation of 100 Mb/s switches

· Upgrade for large installed base of 10/100 Ethernet

· Upgrade for large installed base of Category 5 cabling and Class D

links

Market surveys indicate that Category 5 balanced copper cabling is

the predominant installed intra-building horizontal networking

media today (see # 5 below.) The market acceptance of 100BASE-TX

is a clear indication that copper cabling will continue to be the

medium of choice wherever it can be applied. 1000BASE-T is the

natural extension of this evolution and can expect broad market

acceptance as the demand for network speed increases. 1000BASE-T

will continue the Ethernet tradition of providing balanced cost

solutions.

Interest in 1000BASE-T has been demonstrated by the attendance of

more than fifty vendor and user representatives at technical

meetings at the November '96 Plenary, attendance at a subsequent

Interim meetings, and by participation in an email forum devoted to

facilitating technical development in this area. More than 29 vendor

companies have indicated that they plan to participate in the

standardization of 1000BASE-T. This level of commitment indicates

that the standard will be supported by multiple vendors, and that

there will be a wide variety of equipment available to support

gigabit speed applications copper links.

=============================================================

2. Compatibility with IEEE Standard 802.3

· Conformance with CSMA/CD MAC, PLS

· Conformance with 802.2

· Conformance with 802 FR

------------------------------------------------------------------------

The proposed standard will conform to the CSMA/CD MAC with

currently authorized extensions, appropriately adapted for 1000

Mb/s use.

In a fashion similar to the 100BASE-T standard, the current physical

layers will be replaced with new Physical Layers (PHY) as

appropriate for 1000 Mb/s operation over the defined link: a link

which meets the link requirements of 4 pair Category 5 100 ohm

balanced copper component specifications as specified in

ANSI/TIA/EIA-568-A or ISO/IEC 11801 : 1995 and the channel

specifications of TIA/EIA568A Annex E. 1000BASE-T offers the

maximum compatibility with the current installed base of more than

60 million CSMA/CD nodes, most of which copper cabling systems.

Support of 802.3 Auto-Negotiation will ensure that 802.3  UTP

solutions continue to be auto-configuring.

The Management Information Base (MIB) for 1000BASE-T will

maintain consistency with the current 802.3 MIB for 10/100 Mb/s

operation, allowing a consistent management model across all

operating speeds.

Conformance with 802.2 is provided by the overlying 802.3z MAC

sublayer.

The proposed standard will conform to the 802 Functional

Requirements Document, with the possible exception of the Hamming

distance.

The 1000BASE-T PHY will conform to the Gigabit Media Independent

Interface (GMII) to be specified in 802.3z.

Two kilometer network spans, while not supported specifically by

1000BASE-T, will be supported by combination with other members

of the 802.3 family of 1000 Mb/s standards.

=============================================================

3. Distinct Identity

· Substantially different from other 802.3 specs/solutions

· Unique solution for problem

· Easy for document reader to select relevant spec

-----------------------------------------------------------------------

The proposed standard is a 1000Mb/s upgrade for 802.3 users based

on the 802.3 CSMA/CD MAC.

It will be the only balanced copper solution for 1000 Mb/s capable of

providing service over the defined link. As such, it offers an easy

upgrade for the 1000s of users who have installed Category 5 cable

plants.

It is substantially different from other 802.3 copper solutions in that

it supports 1 Gb/s operation over the defined link.

The proposed standard will be a supplement to the existing 802.3

standard and supplements in progress and will be formatted as a

new clause, making it easy for the reader to select the relevant

specification.

==========================================================

4. Technical Feasibility

· Demonstrated feasibility

· Proven technology

· Confidence in reliability

------------------------------------------------------------------------

Technical presentations given to 802 have demonstrated the

feasibility of using the digital signal processing technology in

Category 5/Class D UTP network topologies at a rate of 1000 Mb/s.

These solutions build on current 802.3 technologies, including

100BASE-TX and 100BASE-T2, and on technologies currently in use

in other data communications areas.

In spite of the increased data rate, the signaling bandwidth required

of each of the line coding implementations currently under

consideration for 1000BASE-T is no greater than that required for

100BASE-TX. All of the line codes proposed have been simulated in

mathematical models that have been verified by actual

measurements and are widely accepted by the field as being a valid

indicator of real life performance. Significant work has been done

testing line code proposals over the defined link using simulation and

actual cable links.

Experience with equipment using ADSL, HDSL and QAM based

signaling techniques in the field has demonstrated the reliability of

signaling techniques such as those proposed by 1000BASE-T. Similar

modulation systems are widely used in cable modems, hard disk

drives, etc.

==========================================================

5. Economic Feasibility

· Cost factors known, reliable data

· Reasonable cost for performance expected

· Total installation costs considered

------------------------------------------------------------------------

The line code solutions proposed for 1000BASE-T are all based on

existing data communications solution technologies.

Although an accurate cost model cannot be established until the line

code solution has been selected, projections indicate that the

cost/performance ratio for 1000BASE-T vis-à-vis 100BASE-TX will

be about the same as that offered by 100BASE-TX vis-a-vis 10BASE-

T at the time of initial introduction (1994.) The cost of proposed

implementations are expected to be 4-6X that of 100BASE-TX. Given

consistent progress on the standard, initial silicon implementations

should be available in 1H98.

Customers will be able to use existing Category 5 balanced copper

cabling that conforms to the defined link. The cost model for

horizontal copper cabling is well established.

A variety of surveys conducted over the past five years have

demonstrated that Category 5 cabling is the dominant cabling in

place today. Payne (92) found that 65% of the 117 companies

surveyed had Category 5 cabling as a corporate standard and

projected that the number would grow to 83% in two years (1994.) A

Sage Research survey (95) found that 70.5 percent of surveyed

companies with more than 1000 employees used Category 5 cabling

for horizontal links.

For companies with installed Category 5 wiring, 1000BASE-T offers

the lowest cost Gigabit solution; all upgrade costs will reside in the

silicon and magnetic components, not the link.