# IEEE 802.9
 Wireless IS LAN MAC and PHY

## IEEE 802 Tutorial
Tuesday evening -- July 8, 1997

Chandos A. Rypinski, LF
LACE, Inc. and Ubiquity Communication, Inc.

# Introduction -- Part I

## Goal:  New Wireless IS LAN MAC and PHY for 802.9 IS LAN
(IS = Integrated Services)

## Use of new NII Band frequency space--200 MHz at 5.25 GHz

## Consider for differentiation:

### World-wide developments of Wireless ATM

### Completion of IEEE 802.11 Standard

## Action Request--write/message to Dhadesugoor Vaman, the Chairman of 802.9:
 I / we support the formation of a Study Group to address the possibility of a PAR for a Wireless MAC and PHY for 802.9.  Depending upon the direction taken, it is probable that (my organization) would participate in an effort to develop such a standard.

# Service Definition at User Station

## Packet LAN services

### 802 LLC higher layer interface, and at this point:

### Successful transfers of errorless packets at wireline probability

### Low access delay (<1.5 msec) and high transfer rate after access (16 Mbps)

## Connection-type services

### ISDN/Telecom-based higher layers with on-demand up to 24/30 B channels one or two-way

### Future capacity reservation

### Low through-system transfer delay (<6 msec including quantization delay)

# System Suitability Definition

## Near 100% area coverage within a building and nearly that within a campus

## Intensive reuse of the channels for high Mbps per hectare/acre

### Interference-limited radio system design

### Robust rather than high bps/Hz radio modulations

## One radio infrastructure for all supported services

# External WAN Interconnectability Supported

## Telecom ISDN (PRI) and customary transports (T-1, T-3, SONET)

## Router Ports for 100BaseT and possibly FDDI

## ATM UNI at selected transport interfaces (future)

# Markets and Applications

## In and out-of-building access to both external LAN, internet and telecom networks where ever that function is useful

### School, university, commercial and manufacturing campii or individual premises

### Wireless gateways to any existing private or public network

### Fast setup facilities for convention centers, disasters or construction sites

# Value of Extending 802.9

## 802.9 is the primary Integrated Services group in 802 at workstation interface level

## There is no difference in the services provided by 802.9 now relative to those of a new PAR

## 802.9 has defined an interface between a user station and a physical medium where a central manager serves multiple physical medium ports

## 802.9 has already dealt with localizing the standard to a station interface when there is a multiport central machine external to the scope of the standard

## 802.9 is more likely to recognize need for an ATM upward interface than pure LAN groups

# Why Not Extend 802.11?

## 802.11 has decided that their use of the NII band will use their existing MAC including distributed logic and LBT, but with modifications for increased medium transfer rate.

## 802.11 has no provision for telecom connection-type services

## 802.11 has no provision for central coordination of many channel sharing access points

## In this radio band, a variant of HIPERLAN /1 is an existing standard; and it might be a better MAC for an LBT access method with distributed logic

# Do WAND and HIPERLAN /2 make this project redundant?

## WAND (Wireless ATM Network Demonstrator) is the European wireless ATM development.  ETSI RES 10 HIPERLAN /2 is the development of the physical layer.

## The headline statement will be true when and if PC’s commonly execute ATM setup logic.  Winsock 2 is a step in this direction.  When the general purpose PBX’s use an ATM rather than an ISDN interface, it is here.

## The proposal is that stations use legacy protocol stacks, and that the formal transition to ATM, when required, comes at a central hub equipment

# Request for Action:

## Address the Chairman of IEEE 802.9:
	Professor Dhadesugoor Vaman               <vaman@ati.stevens-tech.edu>

## In writing use words like the following:

I / we support the formation of a Study Group to address the possibility of a PAR for a Wireless MAC and PHY for 802.9.  Depending upon the direction taken, it is probable that (my organization) would participate in an effort to develop such a standard.

Signature and organizational identification

# Technology--Part II

## Segmentation is Inevitable and Beneficial

## The 802.9 Protocol Stack Now (Figure)

## The MAC function should be common to both Packet and Connection bearing cells

## The 802.9 Protocol Stack After Adding a Wireless PHY/MAC (Figure)

## The Local Connection Concept

## Protocol Requirement for 90% Capacity Utilization

## Radio Propagation Problems Influencing MAC

## Important Details

## Anticipation of Changes in NII Band FCC Rules

# Segmentation is Inevitable and Beneficial

## Segmentation to a 48 octet payload corresponds to a 12 millisecond accumulation delay at a 32 Kbps channel rate.

## The access delay experienced for packets and connections is a multiple of the average holding time per channel use

## 802.11 “fragments” packets to 256 octets for transmission

## The shorter the segment the higher the probability of error-free reception and the less channel time invested in unusable transmission--and the higher the overhead

## For peer-to-peer communication where peers may or may not be served by the same access point, no reassembly of segments is necessary for local relay

# The 802.9 Protocol Stack Now

## There are separate LLC and MAC’s for packet and connection type services

## The two stacks are use separate multiplexed space in the PHY frame

## The modem and line functions are common to both

# A Common MAC Function for Both Packet and Connection Bearing Cells

## All of the present IS Protocols (802.6, 802.9 and Telecom SDH) are an isochronous medium where time slots are dedicated to either packets or connections using a physical medium multiplexer

## Priority and queue management are more efficient using a common space rather than a fixed partition into one space for each service

## The total amount of logic equipment is reduced by shared rather than independent MAC’s

# The 802.9 Protocol Stack After Adding a Wireless PHY/MAC

## There are still two LLC and higher level interfaces for packets and connections

## The two services are separately segmented, multiplexed and sent as a common stream to the MAC

## There is now one common MAC and PHY

# The Local Connection Concept

## Stations talk to the central Hub to request service and use of the radio medium

## After setup, station-originated connections and packets are repeated either to a second connection/path to an external network or to another station within the system

## Segmented packets are treated as a connection with a duration of one packet length between station and Hub

## Connection type services include bounded size reservation of future capacity managed by the Hub where the reservation is not that of a particular time slot

# Protocol Requirement for 90% Capacity Utilization

## To achieve near 100% of capacity utilization, it is necessary that access for service requests be near out-of-band and near non-blocking

## The function must be that of Erlang C blocking to aid in calculation of probable wait delay as a function of traffic loading

## The delay experienced for both packets and cells is a multiple of the average holding time for each individual use of the channel--short bursts are essential to low access delay

## The channel(s) must be as large as possible for a given pool of users

# Radio Propagation Problems Influencing MAC

## High inherent non-random error rate from multipath and interference

## Variability of user station location and resulting propagation path

## Need for access point diversity requires suspension of principle:
 “only one path at a time to any user station”

## Variable path to user station must be shielded from outside networks

# Important Details

## Forward error correction

## MAC level repeat send

## Positive ACK for all packet segments

## NACK and one retry for connection segments

## Modulation method optimized for resistance to like-signal interference

## Transmitters are ON only when in use

# Anticipation of Changes in NII Band FCC Rules

## The WINForum SRDS Committee is working on sharing rules

## Results will be incorporated in a future filing with FCC

### It is improbable that LBT will be either required or prohibited

### Probably, there will be defined channeling 20-25 MHz

### There might be duty cycle or transmitter ON time limitations

### There will almost surely be additional restrictions on out-of-band radiation