# ZyXEL ES-3124PWR V3.60(TY.4)C0

# Release Note/Manual Supplement

Date:  Apr. 25, 2004

This document describes the features in the ES-3124PWR product for its 3.60(TY.4)C0 release.

## Support Platforms:

ZyXEL ES-3124PWR V3.60(TY.4)C0 supports models: ZyXEL ES-3124PWR

## Version:

OS Version: V3.60(TY.4) | 04/25/2006 14:43:18

BootBase Version: V0.1 | 02/01/2005 16:33:32

## Features:

- 24 Auto MDI/MDI-X 100BASE-TX ports with power over Ethernet support

- 2 1000BASE-T ports for cascading

- 2 1000BASE-T/SFP combo ports for uplink

- 16K layer2 MAC addresses table

- Support 32MB external packet buffer.

- IEEE 802.1D transparent bridging

- Port-based VLAN

- IEEE 802.1Q tag-based VLAN

- IGMP snooping

- MAC filtering

- Management through console, telnet, SNMP or web management

- Firmware upgrade by FTP

- Configuration saving and retrieving

- Overheat detection

- LED indications for link status

- External power backup for internal power module failure

- 9K jumbo frame support

- Filtering/Mirroring by L2/L3/L4 rules

- Bandwidth control by L2/L3/L4 rules

- Egress traffic shaping per port at 64Kbps step

- Private VLAN for port isolation

- Double tagging

- BPDU transparency.

- SSHv1/SSHv2/SSL

- Integrated multicast services.

- Dual rate three colors.

- ZyXEL EMS support.

- Power management on PoE ports

## Known Issues:

- Management port test fail in 100M long cable.

- Management port has compatible issue with Cnet Pro 2000 network interface card.

- The packets which destination address is marked in the filter setting will always send to port 1.

- Symbol and dribble error packets will count into FCS error counter.

- Filtering: In some special cases, setting one MAC with discard-source will cause device filter all packets with this MAC, ether source MAC or destination MAC.

- Do not suggest cooperate LACP with RSTP more than ten ports.

- Do not support tagged frame forwarding in the Port-Based VLAN.

- If total PD power consumption is over power supply specification, the latter plugging PD wouldn’t be powered over Ethernet.

- If the contact between RJ45 and wire cable or the wire cable is unstable, the cable diagnostics maybe fail.

## Fix:

N/A

## Change History:

V3.60(TY.4) (04/25/2006)

Modify the high threshold of system FAN RPM to 7000

## Manufacturing Data:

ZyNOS Version          : V3.60(TY.4) | 04/25/2006 14:43:18

Bootbase Version       : V0.1 | 02/01/2005 16:33:32

Serial Number          : xxxxxxxxxxxxxxx

Vendor Name            : ZyXEL Communications Corp.

Product Model          : ES-3124PWR

ZyNOS Code Model       : ES3124PWR

HTP Code Model         : Unknown

ZyNOS ROM address      : 50080000

System Type            : 10

MAC Address            : 001349000001

Default Country Code   : FF

Boot Module Debug Flag : 01

RomFile Version        : 84

RomFile Checksum       : ae79

ZyNOS Checksum         : 868b

SNMP MIB level & OID   : 060102030405060708091011121314151617181920

Main Feature Bits      : C0

Other Feature Bits     :

D1 00 00 00 00 00 00 00-00 00 00 00 00 00 00 00

00 00 00 00 00 00 00 00-00 41 13 00 00 00

## Firmware Upgrade:

The ES-3124PWR uses FTP to upgrade firmware in run-time through its built-in FTP server.  You can use any FTP client (for example, ftp.exe in Windows) to upgrade ES-3124PWR. The upgrade procedure is as follows:

Upgrade ES-3124PWR FW:

C:\> ftp <ES-3124PWR IP address>

User : admin

Password: 1234

230 Logged in

ftp> put 360TY4C0.bin ras

ftp> bye

Where

- User name : the management user name, admin by default

- Password : the management password, 1234 by default

- 360TY4C0.bin : the name of firmware file you want to upgrade

- ras : the internal firmware name in ES-3124PWR

## Configuration Upgrade:

The ES-3124PWR uses FTP to upgrade configuration in run-time through its built-in FTP server.  You can use any FTP client (for example, ftp.exe in Windows) to upgrade ES-3124PWR. The upgrade procedure is as follows:

Upgrade ES-3124PWR configuration:

C:\> ftp <ES-3124PWR IP address>

User : admin

Password: 1234

230 Logged in

ftp> put 360TY4C0.rom rom-0

ftp> bye

Where

- User name : the management user name, admin by default

- Password : the management password, 1234 by default

- 360TY4C0.rom : the name of configuration file you want to upgrade

- rom-0 : the internal configuration name in ES-3124PWR